Python for S60

Sensor module for S60 Sensor FW

Table of Contents

1. Introduction	3

sensor -- Module to access the device sensors	3

2. Module Level Functions	4

2.1 List Channels	4

Function signature : list_channels()	4

2.2 Query logical name:	4

Function signature : get_logicalname(<DataLookupClass>, value)	4

3. Classes	5

3.1 Base class	5

3.1.1 Object creation	5

Function signature : __init__([data_filter=None])	5

3.1.2 Set Data and Error Callback:	5

Function signature : set_callback(data_callback, [error_callback=None])	5

3.1.3 Open and listen	5

Function signature : start_listening()	5

3.1.4 Stop and close	5

Function signature : stop_listening()	5

3.1.5 Set/Get sensor channel property	6

3.1.6 Class attributes	6

3.2 class AccelerometerXYZAxisData	7

3.3 class AccelerometerDoubleTappingData	8

3.3.1 Set/Get property	8

3.4 class MagnetometerXYZAxisData	10

3.5 class MagneticNorthData	12

3.6 class AmbientLightData	13

3.7 class ProximityMonitor	14

3.8 class OrientationData	15

3.9 class RotationData	16

- 1. Introduction

- sensor -- Module to access the device sensors

This PyS60 sensor module supports accessing the sensors on the devices that have S60 Sensor Framework libraries. The S60 Sensor Framework was introduced in S60 5th Edition, but is also backported to S60 3rd Edition, Feature Pack 2 for some mobile devices, as well as the Nokia E66 device, which is an S60 3rd Edition, Feature Pack 1 device with sensor APIs based on the S60 Sensor Framework.

The sensor module offers direct access to a device's physical sensors. The following sensor channels are supported by the sensor module, provided the device supports them:

- Accelerometer XYZ sensor channel

- Rotation sensor channel

- Orientation sensor channel

- Accelerometer double-tap sensor channel

- Proximity monitor sensor channel

- Ambient light sensor channel

- Magnetic North sensor channel

- Magnetometer XYZ sensor channel

The following table lists the sensors available on different S60 devices

|  | Accelerometer Double Tap | Accelerometer XYZ | Orientation | Rotation | Ambient Light | Magnetic North | Proximity Monitor | Magnetometer XYZ | S60 platform |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N85 | x | x | x | x |  |  |  |  | 3rdFP2 |
| E66 | x | x | x | x |  |  |  |  | 3rdFP1 |
| N96 | x | x | x | x |  |  |  |  | 3rdFP2 |
| E75 | x | x | x | x |  |  |  |  | 3rdFP2 |
| 6720 |  | x | x |  | x | x |  |  | 3rdFP2 |
| 5800 | x | x | x | x | x |  | x |  | 5thEd |
| 6210 | x | x | x | x |  | x |  | x | 3rdFP2 |
| 6710 | x | x | x | x |  | x |  | x | 3rdFP2 |
| E55 | x | x |  | x | x | x |  | x | 3rdFP2 |

These sensors are mapped to a class using which the sensor channel data can be accessed. To access a particular sensor data an object of the respective class has to be created first. Then the data callback function can be set using the set_callback() method. To start and stop receiving updates the start_listening() and stop_listening() methods can be used.

Note: This module's API compatibility between 1.9.x  versions isn't guaranteed until 2.0. It won't be broken without good reason,  but it can happen.

- 2. Module Level Functions

- 2.1 List Channels

- Function signature : list_channels()

This returns a list of dictionaries containing all the available sensors on the device. The returned dictionary has the following format:

[

{'id': channel id, 'type': channel type, 'name’: channel name}

{'id': channel id, 'type': channel type, 'name’: channel name}

….

]

where channel_id, channel_type and channel_name will have the respective channel’s values as strings.

- 2.2 Query logical name:

- Function signature : get_logicalname(<DataLookupClass>, value)

This function can be used for querying the logical name based on value. The file sensor_defs.py has the mapping of different sensor properties to their respective hex/decimal values. Following table contains the sensor classes, supported by get_logicalname() and the respective data lookup classes.

| Sensor Class | DataLookupClass |
| --- | --- |
| ProximityMonitor | TProximityState |
| OrientationData | TSensrvDeviceOrientation |
| AmbientLightData | TSensrvAmbientLightData |
| AccelerometerDoubleTappingData | KSensrvAccelerometerDirection |

- 3. Classes

- 3.1 Base class

The base class to all types of sensor class is _Sensor. This class provides the methods – set_callback, start_listening and stop_listening which are common to all the sensor class objects. The individual sensor class objects should be used to use a specific sensor.

- 3.1.1 Object creation

- Function signature : __init__([data_filter=None])

The data_filter argument is only applicable for *XYZAxisData and RotationData sensor classes.

Possible Values:  MedianFilter(), LowPassFilter()

- If nothing is passed then the data is left as-is without any filtering

- MedianFilter & LowPassFilter are standard noise filtering algorithms which provide a smoother form of a signal removing the short-term oscillations, leaving only the long-term trend.

- 3.1.2 Set Data and Error Callback:

- Function signature : set_callback(data_callback, [error_callback=None])

Sets the data and error callback function. The error callback function will get an argument which will contain a map with Channel ID and error string. The data callback function set will not be passed any arguments.

- 3.1.3 Open and listen

- Function signature : start_listening()

Opens the sensor channel and start listening. Returns True on success and False on failure.

- 3.1.4 Stop and close

- Function signature : stop_listening()

Stop listening to the open channel and close the channel. To start receiving updates again the start_listening method can be called on the same sensor object.

- 3.1.5 Set/Get sensor channel property

The current release of PyS60 sensor module does not support either retrieving or modifying all the sensor properties of a particular channel like DataRate, MeasureRange, ScaledRange etc. This feature will be provided in the future dot releases. Refer the individual sensor class description for more details.

- 3.1.6 Class attributes

The sensor classes have one or more attributes which will contain the data returned by the respective sensor. These attributes will be set before the registered data callback function is called and can be accessed using the respective sensor class object.

- 3.2 class AccelerometerXYZAxisData

- Detects movement gestures, such as moving the device up or down

- Inherits from the _Sensor base class

Class attributes:

- x : X-axis value

- y : Y-axis value

- z : Z-axis value

Example:

from sensor import *

import e32

import time

class DemoApp():

def __init__(self):

self.accelerometer = \

AccelerometerXYZAxisData(data_filter=LowPassFilter())

self.accelerometer.set_callback(data_callback=self.my_callback)

self.counter = 0

def my_callback(self):

# For stream sensor data the callback is hit 35 times per sec(On

# 5800). The device cannot handle resource hungry operations like

# print in the callback function for such high frequencies. A

# workaround is to sample the data as demonstrated below.

if self.counter % 5 == 0:

print  "X:%s, Y:%s, Z:%s" % (self.accelerometer.x,

self.accelerometer.y, self.accelerometer.z)

self.counter = self.counter + 1

def run(self):

self.accelerometer.start_listening()

if __name__ == '__main__':

d = DemoApp()

d.run()

e32.ao_sleep(1)

d.accelerometer.stop_listening()

print "Exiting Accelorometer"

- 3.3 class AccelerometerDoubleTappingData

- Detects a double-tap on the device where the taps occur in quick succession and in the same direction.

- Inherits from the _Sensor base class

Class attribute:

direction : Hex value indicating the tap direction. Using get_logicalname API and classname as KSensrvAccelerometerDirection the direction can be determined in human readable form.

- 3.3.1 Set/Get property

This sensor class provides additional functions which can be used to set/get some of the properties specific to this sensor. NOTE: set/get sensor property API is not complete yet and is not supported for all sensors or all types of properties. These APIs might change in the future dot releases.

get_axis_active() Returns x, y, z values: 1 if axis is active else 0.

set_axis_active([x=None, y=None, z=None])  Sets one or more axis as active. Pass 1 to set the axis and 0 to disable it.

get_properties() Returns TapThresholdValue, TapDurationValue, TapLatencyValue, TapIntervalValue

set_properties([DblTapThresholdValue = None, DblTapDurationValue = None, DblTapLatencyValue = None, DblTapIntervalValue = None]) Sets the tap related properties

Example:

from sensor import *

import e32

class DemoApp():

def __init__(self):

self.doubletap = AccelerometerDoubleTappingData()

self.doubletap.set_axis_active(x=0, y=1, z=1)

print "Active Axis are: ", self.doubletap.get_axis_active()

self.doubletap.set_callback(data_callback=self.my_callback)

def my_callback(self):

print "Raw Direction value", self.doubletap.direction

print "Direction: ", get_logicalname(KSensrvAccelerometerDirection,

self.doubletap.direction)

def run(self):

self.doubletap.start_listening()

if __name__ == '__main__':

d = DemoApp()

d.run()

e32.ao_sleep(15)

d.doubletap.stop_listening()

print "Exiting Double Tap"

- 3.4 class MagnetometerXYZAxisData

- Indicates the strength of the geomagnetic flux density in the X, Y and Z axes.

- Only calibrated axis data is exposed right now and not raw data.

- Inherits from the _Sensor base class

Class attributes:

- x : X-axis value

- y : Y-axis value

- z : Z-axis value

- calib_level: Indicates the calibration level.

  - Possible values:

0 - Not calibrated

1 - Low calibration.

2 - Medium calibration

3 - High accuracy

Example:

from sensor import *

import e32

class DemoApp():

def __init__(self):

self.magnetometer = \

MagnetometerXYZAxisData(data_filter=LowPassFilter())

self.magnetometer.set_callback(data_callback=self.my_callback)

self.counter = 0

def my_callback(self):

# For stream sensor data the callback is hit 35 times per sec(On

# 5800). The device cannot handle resource hungry operations like

# print in the callback function for such high frequencies. A

# workaround is to sample the data as demonstrated below.

if self.counter % 5 == 0:

print  "Calib:", self.magnetometer.calib_level

print "X:%s, Y:%s, Z:%s" % (self.magnetometer.x,

self.magnetometer.y, self.magnetometer.z)

self.counter = self.counter + 1

def run(self):

self.magnetometer.start_listening()

if __name__ == '__main__':

d = DemoApp()

d.run()

e32.ao_sleep(5)

d.magnetometer.stop_listening()

print "Exiting MagnetometerAxis"

- 3.5 class MagneticNorthData

- Indicates the number of degrees between the device and magnetic north

- Inherits from the _Sensor base class

Class attribute:

- azimuth :  0 to 359 clockwise degrees from magnetic north

Example:

from sensor import *

import e32

class DemoApp():

def __init__(self):

self.magnetic_north = MagneticNorthData()

self.magnetic_north.set_callback(data_callback=self.my_callback)

def my_callback(self):

if self.magnetic_north.calib_level > 0:

azimuth = str(self.magnetic_north.azimuth)

print "calibration level", self.magnetic_north.calib_level

print "azimuth", azimuth

def run(self):

self.magnetic_north.start_listening()

if __name__ == '__main__':

d = DemoApp()

d.run()

e32.ao_sleep(5)

d.magnetic_north.stop_listening()

print "Exiting MagneticNorth"

- 3.6 class AmbientLightData

- Indicates the current light level

- Inherits from the _Sensor base class

Class attribute:

- ambient_light : 0 to 100 percent light. Use get_logicalname API with classname as TSensrvAmbientLightData to get the logical names.

Example:

from sensor import *

import e32

class DemoApp():

def __init__(self):

self.ALS = AmbientLightData()

self.ALS.set_callback(data_callback=self.my_callback)

def my_callback(self):

print 'ALS : ', get_logicalname(TSensrvAmbientLightData,

self.ALS.ambient_light)

def run(self):

self.ALS.start_listening()

if __name__ == '__main__':

d = DemoApp()

d.run()

e32.ao_sleep(30)

d.ALS.stop_listening()

print "Exiting Ambient Light"

- 3.7 class ProximityMonitor

- Indicates how close the device is to, for example, the user's hand or ear

- Inherits from the _Sensor base class

Class attribute:

- proximity_state : The possible values are 0, 1 and 2. Use get_logicalname API with TProximityState as the class name to get the logical names of these values.

Example:

from sensor import *

import e32

class DemoApp():

def __init__(self):

self.proxi = ProximityMonitor()

self.proxi.set_callback(data_callback=self.my_callback)

def my_callback(self):

print 'proxi : ', get_logicalname(TProximityState,

self.proxi.proximity_state)

def run(self):

self.proxi.start_listening()

if __name__ == '__main__':

d = DemoApp()

d.run()

e32.ao_sleep(10)

d.proxi.stop_listening()

print "After Stop Listening"

e32.ao_sleep(5)

print "Exiting Proximity"

- 3.8 class OrientationData

- Indicates the orientation of the device, for example: display up or display down

- Inherits from the _Sensor base class

Class attribute:

- device_orientation : Values range from -1 to 6. To determine the logical names of these values get_logicalname API can be used with classname as TSensrvDeviceOrientation.

Example:

from sensor import *

import e32

class DemoApp():

def __init__(self):

self.orientation = OrientationData()

self.orientation.set_callback(data_callback=self.my_callback)

def my_callback(self):

print 'orientation : ', get_logicalname(TSensrvDeviceOrientation,

self.orientation.device_orientation)

def run(self):

self.orientation.start_listening()

if __name__ == '__main__':

d = DemoApp()

d.run()

e32.ao_sleep(10)

d.orientation.stop_listening()

print "Exiting Orientation"

- 3.9 class RotationData

- Detects the rotation of the device about each axis

- Inherits from the _Sensor base class

Class attributes:

- x : X-axis value

- y : Y-axis value

- z : Z-axis value

Example:

from sensor import *

import e32

class DemoApp():

def __init__(self):

self.rotation = RotationData()

self.rotation.set_callback(data_callback=self.my_callback)

self.counter = 0

def my_callback(self):

# For stream sensor data the callback is hit approximately 20

# times per sec(On 5800). The device cannot handle resource

# hungry operations like print in the callback function for such

# high frequencies. A workaround is to sample the data as

# demonstrated below.

if self.counter % 5 == 0:

print "X:%s, Y:%s, Z:%s" % (self.rotation.x,

self.rotation.y, self.rotation.z)

self.counter = self.counter + 1

def run(self):

self.rotation.start_listening()

if __name__ == '__main__':

d = DemoApp()

d.run()

e32.ao_sleep(5)

d.rotation.stop_listening()

print "Exiting Rotation"