- S60 3.2

- SUPL PSY

- Design Document

| Team name | Author |
| --- | --- |
| Location Technology Team | Marko A. Alanen |

- Change history:

| Version | Date | Status | Comments |
| --- | --- | --- | --- |
| 3.0 | 01.06.2007 | Approved | Incorporated review comments, changes to add filter for accuracy received in the position information. |
| 2.0 | 21.04.2006 | Approved |  |
| 1.0 | 20.01.2006 | Approved | Initial version |

- Table of contents:

1.	Document control	5

1.1	References	5

1.2	Documentation conventions	5

1.3	Abbreviations	5

1.4	Definitions	6

2.	General	7

2.1	Introduction	7

2.2	Subsystem environment	7

2.3	Services required	8

2.3.1	From the Location Framework subsystems	8

2.3.2	From the Generic Positioning Plug-ins subsystem	8

2.3.3	From the application user interfaces subsystems	8

2.3.4	From the Symbian OS subsystems	8

2.3.5	From the Domestic OS subsystems	9

2.3.6	From HW	9

2.4	Services provided	9

2.4.1	SW IF	9

3.	Subsystem overview	10

3.1	Design	10

3.1.1	Architectural requirements	10

3.1.2	Component diagrams	10

3.1.2.1	Subsystem	10

3.1.3	Component descriptions	11

3.1.3.1	SUPL PSY	11

3.1.3.2	Positioning Plug-in API	11

3.1.3.3	SUPL PSY Information API	11

3.1.3.4	SUPL Terminal Initiation API	11

3.1.4	State diagrams	11

3.1.5	Variability	11

3.1.6	Flag definitions	11

3.1.7	Operations	12

3.1.7.1	PSY loading	12

3.1.7.2	PSY unloading	12

3.1.7.3	Requesting location	13

3.1.7.4	Canceling a location request	15

3.2	System resource usage	16

3.2.1	Files	16

3.2.2	Databases	16

3.2.3	Memory consumption	16

3.2.4	Task allocation	16

3.2.5	Timers	16

3.3	Security	16

3.3.1	Capabilities of this component	16

3.3.2	Input and output validation	16

3.3.3	Environmental assumptions	16

3.3.4	Additional enforcements	16

3.4	Exception handling	16

4.	Subsystem Usage	17

4.1	Building	17

4.1.1	Variation and feature flags	17

4.1.2	Localization	17

4.2	User's guide	17

4.2.1	Developers	17

4.2.1.1	Debugging	17

4.2.2	End users	17

5.	SUPL PSY component	18

5.1	Design	18

5.1.1	Class descriptions	19

5.1.2	Operations	20

5.1.2.1	PSY loading	20

5.1.2.2	PSY unloading	21

5.1.2.3	Location request	22

5.1.2.4	Canceling a location request	24

5.2	System resource usage	25

5.2.1	File structures	25

5.2.2	Database structures	25

5.2.3	Timers and active objects	26

5.3	Exception handling	26

Appendix A: Location request error code	27

# Document control

## References

- S60 Location Framework Subsystem Design Document

- S60 Positioning Plug-in API Specification Document

- S60 Generic Positioning Plug-ins Subsystem Architecture Description

- S60 SUPL Subsystem Design Document

- S60 SUPL Terminal Initiation API Specification Document

- S60 Network Positioning Proxy Design Document

## Documentation conventions

Code is shown with the Courier New font.

## Abbreviations

| FW | Framework |
| --- | --- |
| MLFW | Mobile Location Framework |
| OEM | Original Equipment Manufacturer |
| PSY | Positioning System (In the context of this document, also means a Positioning Plug-in to MLFW.) |
| ROM | Read-Only Memory |
| SET | SUPL Enabled Terminal |
| SLC | SUPL Location Center |
| SLP | SUPL Location Platform |
| SPC | SUPL Positioning Center |
| SUPL | Secure User Plane Location |

## Definitions

| ECom | Symbian OS framework for plug-in DLLs. |
| --- | --- |
| Fix | Information received from SUPL subsystem about the current location. |
| Max age | The maximum age of the fix that can be delivered to the requestor. |
| Request bundling | The method for serving multiple simultaneous location requests. All the simultaneous requests are completed with the same fix. |
| Request handler | The singleton that routes the location requests from multiple clients (multiple PSY instances) to the single SUPL subsystem session. |
| Request handling | The process of handling multiple location requests from a single SUPL subsystem session. |
| Tracking | The process of delivering location fixes to the client continuously at a given interval. |

# General

## Introduction

SUPL PSY is a positioning technology plug-in for Location FW. It uses services provided by SUPL subsystem (see reference document [4]) The SUPL subsystem offers general service to use different user plane location protocols defined by different standardization bodies The SUPL subsystem behaves like umbrella that hides differences between SUPL implementations. Note that SUPL PSY is hidden from the clients and system UI, and it is meant to be used only via Network Positioning Proxy (see reference document [6]). The main task of SUPL PSY is to forward a request from the Location FW and Network Positioning Proxy to the SUPL system to obtain location and forward response back to the requestor.  The SUPL PSY does filtering of the location fix returned by SUPL system if the accuracy is present in the location fix.  If the horizontal accuracy in the location fix is worse than the configured accuracy, SUPL PSY will returns error and fix is ignored.  With this feature, SUPL PSY will filter the position fix which may be inaccurate.  For ex., in case of Cell ID based positioning, if the fix is received based on country code instead of cell info with large horizontal accuracy value, this can be filtered in the SUPL PSY.

This document describes the design of SUPL PSY. It is valid from S60 3.1u onwards.

## Subsystem environment

The subsystem environment is shown in Figure 2.1. The SUPL PSY belongs to the Generic Positioning Plug-ins subsystem.

Figure 2.1	Subsystem environment of Generic Positioning Plug-ins subsystem

## Services required

### From the Location Framework subsystems

SUPL PSY uses the Positioning Plug-in API (see reference document [2]), which is exported by the Location Framework subsystem, as a base for its implementation. This enables Network Positioning Proxy to detect and communicate with SUPL PSY.

SUPL PSY publishes its static information through Positioning Plug-in Information API (Central Repository).

### From the Generic Positioning Plug-ins subsystem

SUPL PSY is hidden from the clients and system UI. Only Network Positioning Proxy can use it. Location requests are received via Network Positioning Proxy.

### From the application user interfaces subsystems

There is no direct connection to the application user interfaces subsystems.

### From the Symbian OS subsystems

There are dependencies to the following subsystems:

| Base | Symbian Base is used for basic Symbian OS functionality support. |
| --- | --- |
| System Libraries | SUPL PSY uses Central Repository API to store static information of SUPL PSY. |

### From the Domestic OS subsystems

This component has no direct interfaces to the Domestic OS side subsystems.

### From HW

This component has no direct HW interfaces.

## Services provided

### SW IF

The PSY implements the Positioning Plug-in API and Positioning Plug-in Information API (see reference document [2]).

# Subsystem overview

## Design

### Architectural requirements

Some important architectural considerations for the SUPL PSY are:

- SUPL PSY is implemented as an ECom plug-in. This is required so that SUPL PSY can be detected and used by Network Positioning Proxy.

- SUPL PSY must be defined as hidden PSY. So end user will see only network based positioning method and does not need to understand technique details behind each specific method.

- SUPL PSY must utilize the maximum age parameter related to location requests. The maximum age parameter is used to reuse a previously retrieved fix if the fix is within the maximum age time limit.

- SUPL PSY must support multiple simultaneous client sessions, which means that a position request may be received before the previous one has completed. Request bundling is to be used to deliver the result to the clients when the acquisition is complete.

### Component diagrams

#### Subsystem

The component diagram for the SUPL PSY is shown in Figure 3.2.

Figure 3.2	SUPL PSY component diagram

### Component descriptions

#### SUPL PSY

The SUPL PSY is a positioning plug-in for Location Framework that enables the usage of SUPL subsystem as a location information provider. Location Framework utilizes SUPL PSY via Network Positioning Proxy. PSY properties are listed in Section 5.2.1.

#### Positioning Plug-in API

The Positioning Plug-in API defines the framework classes needed to implement PSYs for the Location Framework. See reference document [2]

#### SUPL PSY Information API

The SUPL PSY publishes its static properties in Central Repository, as defined by Positioning Plug-in Information API. See reference document [2].

#### SUPL Terminal Initiation API

SUPL PSY uses SUPL Subsystem to retrieve current location information via SUPL Terminal Initiation API. See reference document [5].

### State diagrams

SUPL PSY maintains its state internally. It will not report device status and data quality to any other component.

### Variability

There are no variability flags.

### Flag definitions

The table below specifies the R&D flags (debug etc. flags) used by the subsystem. Feature flags related to the subsystem are specified in Section 4.1.1.

| Flag | Description |
| --- | --- |
| __WINS__ | The flag is used to control different implementations of WINS/WINSCW platform and HW platform. Differences are required in some case to make testing on WINS/WINSCW possible. |
| _DEBUG | The flag is used to separate code that is used only in the internal debug build. This concerns the logging for debug purposes. |

### Operations

The SUPL PSY includes only one module and the operations of the module are described in detail in Chapter 5. This section includes only high-level descriptions of the operations.

#### PSY loading

Loading SUPL PSY for the first time will initiate connection to SUPL subsystem.

Figure 3.3	PSY loading

#### PSY unloading

The last PSY instance unloading unloads the whole PSY. This includes the closing the SUPL Terminal Initiation API connection.

Figure 3.4	PSY unloading

#### Requesting location

If the location request cannot be satisfied using a cached fix, SUPL PSY starts requesting location from SUPL subsystem via SUPL Terminal Initiation API. When the fix is received, it then collects the required information and completes the location request.

Figure 3.5	Requesting location

#### Canceling a location request

The canceling of the location request will cancel location request process between SUPL SET and SUPL SLP (see reference document [4]) via SUPL Terminal Initiation API.

Figure 3.6	Canceling of the location request

## System resource usage

### Files

| Filename | Size | Description |
| --- | --- | --- |
| nlasuplpsy.rsc | 80 bytes | Defines ECom plug-in resource file. |
| 10206913.txt | 2050 bytes. | This file is a Central Repository text file defining static information of SUPL PSY |

### Databases

SUPL PSY does not utilize any databases.

### Memory consumption

The ROM usage of this component is 3231B in ARMV5 release. The estimated RAM usage of the PSY is 3 KB. Additional connections are estimated to increase the heap usage by 0.5KB per connection.

### Task allocation

SUPL PSY runs in the Location Server thread and does not allocate any new threads.

### Timers

Timers are presented in Chapter 5.

## Security

### Capabilities of this component

This component is an ECom plug-in, the capabilities describe its level of trust. Capabilities assigned to SUPL PSY are defined in the macro CAP_ECOM_PLUGIN.

### Input and output validation

This component does not require or make any validation for data.

### Environmental assumptions

This component does not make any assumptions about its environment.

### Additional enforcements

This component does not have any additional security enforcements.

## Exception handling

Exception handling is documented in Chapter 5.

# Subsystem Usage

## Building

The table below shows how components map to DLLs and where in the source code tree the project definitions and source code for the different DLLs are located.

| Component name | Directory | MMP file name | DLL name |
| --- | --- | --- | --- |
| SUPL PSY | LocationSuplPsy | group\ nla_suplpsy.MMP | nlasuplpsy.dll |

Building is done by running the following commands in the \group subdirectory of the directory listed in the table:

- bldmake bldfiles

- abld build <TARGET>

Where <TARGET> is one of WINSCW or ARMV5.

To build a SIS-file forARMV5, use the following command in the Install directory:

- makesis nla_suplpsy.pkg

To sign the SIS-file, use the following command in the Install directory:

- signsis nla_suplpsy.pkg nla_suplpsy.sisx rd.cer rd-key.pem

SUPL PSY is not upgradeable. If there is SUPL PSY in ROM, it is not possible to install a signed .sis package on the device.

### Variation and feature flags

SUPL PSY can be included during ROM image creation time by enabling the feature flags __SUPL_FRAMEWORK and __NP_PROXY.

### Localization

This component does not contain any localizable string.

## User's guide

### Developers

#### Debugging

The tracing log of debug builds of the SUPL PSY is generated in the directory C:\logs\suplpsy\log.TXT. Tracing can be switched on and off by creating the directory and removing it, respectively.

### End users

This component is not visible to end-users.

# SUPL PSY component

## Design

This section describes the design of the SUPL PSY.

Figure 5.7 shows the class diagram of SUPL PSY component and its dependencies of Positioning Plug-in API, SUPL Terminal Initiation API, etc. Detailed description of classes is represented in the class descriptions.

Figure 5.7	Class diagram of the SUPL PSY component

### Class descriptions

| Class | Description |
| --- | --- |
| CSuplPsyPositioner | This class provides the interface to Location Framework as specified by Positioning Plug-in API. Each client connection to SUPL PSY makes an instance of this class. |
| CSuplPsyRequestManager | This class provides function to instance a singleton object of SUPL PSY. This class manages all client connections and location requests. |
| CSuplPsyRequestor | This class implements position requestor of SUPL PSY. |
| SuplPsyGenericInfoUser | This is a user class that provides function to handle HPositionGenericInfo. |
| MSuplPsyRequestComplete Listener | This class is request complete listener interface. This class provides the interface for listener to get request complete events. |

### Operations

#### PSY loading

| Operation | PSY loading. |
| --- | --- |
| Preconditions |  |
| Description | The Network Positioning Proxy decides to use SUPL PSY and loads the first instance of the PSY. |
| Inputs |  |
| Modifies |  |
| Outputs |  |
| Postconditions | The PSY is loaded and a connection SUPL Terminal Initiation API is made. Accuracy value is read from the central repository for filtering. |
| Related to | Section 5.1.2.2. |
| Related event |  |
| Exceptions | Out of memory. |

The first PSY loading constructs the CSuplPsyRequestManager singleton object. CSuplPsyRequestManager will then construct CSuplPsyRequestor. CSuplPsyRequestor then connects to SUPL FW through SUPL Terminal Initiation API.  CSuplPsyRequestor constructs the CRepository object and reads allowed accuracy from the central repository.  After reading CRepository object is destroyed.

Figure 5.8	PSY loading

#### PSY unloading

| Operation | PSY unloading. |
| --- | --- |
| Preconditions | Instance of PSY exists. |
| Description | The NPP PSY decides to unload SUPL PSY. If the last instance of CSuplPsyPoistioner is closed, the singleton object CSuplPsyRequestManager will be deleted. Thus, all connections to SUPL FW will be closed. |
| Inputs |  |
| Modifies |  |
| Outputs |  |
| Postconditions | The PSY is unloaded and connection to SUPL FW is closed. |
| Related to | Section 5.1.2.1. |
| Related event |  |
| Exceptions |  |

Figure 5.9	Sequence diagram of PSY unloading when the instance is the only one loaded

#### Location request

| Operation | Location request. |
| --- | --- |
| Preconditions | The PSY is loaded. |
| Description | NPP PSY makes a location request to SUPL PSY. SUPL PSY forwards this location request to SUPL FW if there is not location request currently ongoing. - The information from a cached fix is used if the maximum age parameter allows this. - If the cached fix cannot be used, the new location request is issued to SUPL FW. |
| Inputs | Location request. |
| Modifies | Internal location request status. |
| Outputs | The request result. If the accuracy is present in the location fix, then fix is returned only if the accuracy is within the configured accuracy. If accuracy is not present, fix is returned. |
| Postconditions |  |
| Related to | Section 5.1.2.4. |
| Related event |  |
| Exceptions | Out of memory. |

SUPL PSY requests HPositionGenericInfo from SUPL FW. In addition to normal position information (as TPositionInfo), the following fields, which are supported by SUPL Terminal Initiation API (as defined in reference document [5]), are requested:

- EPositionFieldHorizontalSpeed

- EPositionFieldHorizontalSpeedError

- EPositionFieldVerticalSpeed

- EPositionFieldVerticalSpeedError

Figure 5.10	Sequence diagram for the location request when cached fix could not be returned

Figure 5.11	Sequence diagram for the location request when the cached fix could be returned

#### Canceling a location request

| Operation | Canceling a location request. |
| --- | --- |
| Preconditions | PSY is loaded and a location request is pending. |
| Description | A previously initiated location request is canceled. |
| Inputs |  |
| Modifies | Internal request status. |
| Outputs | The request is completed with KErrCancel. |
| Postconditions | No location request is pending from a client. |
| Related to | Section 5.1.2.3. |
| Related event |  |
| Exceptions |  |

Figure 5.12	Sequence diagram for the canceling of a location request when the canceled location request is the only one

## System resource usage

### File structures

The Central Repository initial value file 10206913.txt contains the SUPL PSY static information (see reference document [2]). Table 5.1 describes the properties that have been set in Central Repository init file.

| Tag | Value |
| --- | --- |
| timetofirstfix | 12.0 (in seconds) |
| timetonextfix | 12.0 (in seconds) |
| horizaccuracy | 200.0 (in meters) |
| vertaccuracy | 0 (in meters) |
| costindicator | ECostPossible |
| powerconsumption | EPowerLow |
| technologytype | KTechnologyNetwork |
| devicelocation | KDeviceInternal |
| capabilities | KCapabilitiesHorizontal |
| classessupported | KPositionClassInfo ¦ KPositionClassGenericInfo |
| allowedaccuracy | 40000 (in meters) |

Table 5.1	PSY properties in 10206913.txt

SUPL PSY is a hidden PSY and does not have a localizable display name.

### Database structures

This component uses no databases.

### Timers and active objects

| Active objects / Timers |  |
| --- | --- |
| CSuplPsyPositioner | Derived from CActive. |

## Exception handling

SUPL PSY will generate panics if it is used incorrectly. The following panic code has been defined. These panics happen only if there is programming bug in Location FW or NPP PSY.

| Panic Reason |  |
| --- | --- |
| EPanicPositionRequest Ongoing | SUPL positioner received location request while there is already a location request ongoing. |
| EPanicPositionerAlready Registered | SUPL positioner has been registered to request manager and try to register to it again. |
| EPanicUnknownPositioning Class | SUPL positioner receive a location request but the positioning class is unknown. |

Refer to Appendix A for a description of error codes of location requests.

# Appendix A: Location request error code

| Error code | Description |
| --- | --- |
| KErrArgument | If class type is not supported. |
| KErrCancel | If location request is canceled successfully. |
| KErrPositionBufferOverflow | If client does not allocate enough buffer to hold the fix. This happens when clients request HPositionGenericInfo. |
| Other system wide error code | If location request is rejected by SUPL FW. |