PDP Contention Management Test Specification

| Security Classification: | Internal - Symbian |
| --- | --- |
| Document Reference: | SGL.XXXnnn.nnn |
| Status: | Draft |
| Version: | 0.2 |
| Last Revised Date: | 22.07.2008 |
| Team/Department : | Networking Services |
| Author(s): | Konstantin Filatov, Alexander Soldatov |
| Owner(s): | Testing |

| Approver(s): | <Test Manager> | Distribution: | <Test Manager> |
| --- | --- | --- | --- |

Contents

1 Introduction	3

1.1 Purpose and Scope	3

2 Test Environment	3

2.1 Overview	3

2.2 Software	3

2.3 Hardware	3

3 Test Cases	4

3.1 CommsDat Configuration	4

3.2 Access Point Priority Inheritance	4

3.3 Contention Management	5

3.4 Progress Notification/Contention Detection 	6

3.5 Availability Notification	7

3.6 Connection Automatic Start	8

4 Further Information	10

4.1 People	10

4.2 References	10

4.3 Open Issues	10

4.4 Glossary 	10

4.5 Document History	10

# Introduction

## Purpose and Scope

The purpose of this document is to specify the series of tests to be used for testing the API and functionality of the PDP Contention Management is realised under PREQ 1927

This test suite is based on TE_EsockTestSteps and therefore consists entirely from a collection of .script and .ini files. This suite is meant to provide a container for test cases related to bearer mobility scenarios.

At the time of writing this document, this suite starts with several bearer independent selection test cases.

# Test Environment

## Overview

The tests are supposed to run both on the emulator and on the hardware.

This test suite consists entirely from a set of scripts to TESTEXECUTE framework. The scripts instruct the framework to load, construct and execute test steps from the TE_EsockTestSteps suite (please refer to TE_EsockTestSteps_Test_Specification document for details).

It's assumed that before every test the CommsDat database is configured to have had five SNAPs (Service Network Access Point) with the PDP Link Layer priorities as show in the table below. The PDP Link Layer priority should be the final priority value after all priority inheritance procedures.

| SNAP # | PDP Link Layer Priority |
| --- | --- |
| 1 | 0 highest priority value, will be changed to 1 after EC120 |
| 2 | 1 |
| 3 | 1 |
| 4 | 7 |
| 5 | KMaxTUint lowest (default) priority value |

Relation between particular SNAP and the SNAP's PDP Link Layer Priority

To make sure in the configuration the every test should recreate the CommsDat by the CED utility with the configuration file applied to this document as a parameter.

## Software

The software environment to run the tests include:

- Windows operation system

- TechView emulator

- TESTEXECUTE framework of the version 3

## Hardware

There is need next hardware to run the tests under emulator environment:

- personal computer

To run the tests under hardware (development board) environment the next equipment will need:

- H6/H4 development board

- H2 development board

- Icera development kit

- Anite development kit

The H6 development board is the main board for testing but it is not ready to fully replace the H4 board so in the case of impossibility to use the H6 the H4 will be used.

The main functionality of the PDP Contention Management will be tested against the H2 development boars combined with Icera and Anite development kits.

# Test Cases

## CommsDat Configuration

| Test No.: | Step ID: | Step Description: |
| --- | --- | --- |
| 1 | pdp-cm-cfg-01-0001 | Can create the CommsDat contained the AP priority filed |
| Actions: | - Run CED with configuration file contains the AP priority field. - Run CEDDUMP to produce the configuration file from the CommsDat taken as result of the previous step execution. - Check that the both configuration file (input for CED and output from CEDDUMP) contain the same values of AP priority fields. |  |
| Expected Result: | The CED and CEDDUMP run without errors. The input and output configuration files contain the same values of AP priority fields. |  |

## Access Point Priority Inheritance

Special CommsDat configuration to implicitly test the priority inheritance.

To ensure the right priority inheritance will be enough to ensure the value propagation from every stack layer to the lower one. For the PREQ 1927 the only interested is the final priority value of the PDP Link layer of the stack. Thus to perform this test the CommsDat should be configured basing on the configuration information from the table below .

| SNAP # | Stack layers priority values |  |  |  |
| --- | --- | --- | --- | --- |
|  | Network | IP Proto | PDP Link | PDP Link Final |
| 1 | max* | average | min* | max |
| 2 | average* | max | min | max |
| 3 | min | average | max | max |
| 4 | average | average | average | average |

* the max priority value is 0

*the min priority value is KmaxTUInt

*the average is any positive numeric value in between 0 and KMaxTUInt

| Test No.: | Step ID: | Step Description: |
| --- | --- | --- |
| 1 | pdp-cm-cfg-xx-yyyy | Check the priority inheritance implicitly by the pre-emption the lower priority connection with the hgher priority one. The test environment should support only one active primary PDP context at the particular moment. |
| Actions: | - Start first connection for the SNAP #4 (to point to particular SNAP use TConnSnapPref parameter) - Start second connection for the SNAP #1 - Close second connections - Restart first connection - Start third connection for the SNAP #2 - Close third connections - Restart first connection - Start fourth connection for the SNAP #3 - Close fourth connections |  |
| Expected Result: | The second, third and fourth connections should be started with no errors because of their priorities. |  |

## Contention Management

Make sure that more prioritized connection pre-empts the less prioritized one.

The CommsDat should be configured agreed the table below

| SNAP # | PDP Link Layer Priority (final value, after inheritance) |
| --- | --- |
| 1 | 0 highest priority value, will be changed to 1 after EC120 |
| 2 | 1 |
| 3 | 1 |
| 4 | 7 |
| 5 | KMaxTUInt lowest / default priority value |

| Test No.: | Step ID: | Step Description: |
| --- | --- | --- |
| 1 | pdp-cm-cfg-xx-yyyy | Ensure the more prioritized connection can pre-empt the less prioritized one and less prioritized connection can not pre-empt the more prioritized connection. The test environment should support only one active primary PDP context at the particular moment. |
| Actions: | - Start first connection C1 for the SNAP #5 (to point to particular SNAP use TConnSnapPref parameter) - Start second connection C2 for the SNAP #4 - Start third connection for C3 the SNAP #3 - Restart connection C2 for the SNAP #4 - Start fourth connection for C4 the SNAP #2 - Start fifth connection C5 for the SNAP #1 |  |
| Expected Result: | Step 2: The connection C1 should be stopped and the connection C2 should be started with no errors. Step 3: The connection C3 should be started with no errors. Step 4: The connection C2::Start(...) should return context activation error code. Step 5: The connection C4::Start(...) should return context activation error code. Step 6:. The connection C5 should be started with no errors. |  |

## Progress Notification/Contention Detection

Make sure that pre-empted connection will be nofied about the reason of pre-emption.

The CommsDat should be configured agreed the table below

| SNAP # | PDP Link Layer Priority (final value, after inheritance) |
| --- | --- |
| 1 | 0 highest priority value, will be changed to 1 after EC120 |
| 2 | 1 |
| 3 | 1 |
| 4 | 7 |
| 5 | KMaxTUInt lowest / default priority value |

| Test No.: | Step ID: | Step Description: |
| --- | --- | --- |
| 1 | pdp-cm-cfg-xx-yyyy | Ensure the pre-empted connection will be notified about pre-emption reason. The test environment should support only one active primary PDP context at the particular moment. |
| Actions: | - Start first connection C1 for the SNAP #4 (to point to particular SNAP use TConnSnapPref parameter) - Subscribe the C1 to ProgressNotification - Start second connection C2 for the SNAP #3 |  |
| Expected Result: | The connection C1 should be stopped and should receive the notification KlinkLayerClosed(KErrConnectionContention). The connection C2 should be started with no errors. |  |

## Availability Notification

Make sure in notification about a PDP context available after closing a connection

The CommsDat should be configured agreed the table below

| SNAP # | PDP Link Layer Priority (final value, after inheritance) |
| --- | --- |
| 1 | 0 highest priority value, will be changed to 1 after EC120 |
| 2 | 1 |
| 3 | 1 |
| 4 | 7 |
| 5 | KMaxTUInt lowest / default priority value |

| Test No.: | Step ID: | Step Description: |
| --- | --- | --- |
| 1 | pdp-cm-cfg-xx-yyyy | Ensure the connection will be notified about availability of the PDP context. The test environment should support only one active primary PDP context at the particular moment. |
| Actions: | - Create connections C1, C2, C3 - Start first connection C1 for the SNAP #5 (to point to particular SNAP use TConnSnapPref parameter) - Subscribe the connections C1, C2, C3 to AvailabilityNotification. The C1 should be subscribed to availability for MBMS TM of SNAP#5, C2 for MBMS TM of SNAP #4 and C3 for MBMS TM of SANP #3 - Start the connection C2 for the SNAP #4 - Start the connection C3 for the SNAP #3 - Stop the connection C3. - Start the connection C1. |  |
| Expected Result: | Step4: The connection C1 should be stopped with no errors. The connection C2 should be started with no errors. The C1 and C3 should receive the “Not Available” notification message. Step 5: The connection C2 should be stopped with no errors. The connection C3 should be started with no errors. The C2 should receive the “Not Available” notification message. Step 6: The connection C3 should be stopped with no errors. The C1 and C2 should receive the “Available” notification message. Step 7: The connection C1 starts with no errors. |  |

## Connection Automatic Start

The PREQ1927 is introduced the new API function RConnection::Start (TConnAutoStartPref). The function gives the application ability to establish the connection when it will be possible instead of to have an error  during the attempt to connecting because all PDP contexts are occupied.

To test this there is need to occupy all possible number of the primary PDP contexts with high priority connections and then try to create one more connection with the less priority using one of occupied PDP context by the function RConnection::Start(TConnAutoStartPref) and then to make sure that the connection will be established only after the hight priority connection on the same PDP context will be stopped.

The CommsDat should be configured agreed the table below

| SNAP # | PDP Link Layer Priority (final value, after inheritance) |
| --- | --- |
| 1 | 0 highest priority value, will be changed to 1 after EC120 |
| 2 | 1 |
| 3 | 1 |
| 4 | 7 |
| 5 | KMaxTUInt lowest / default priority value |

| Test No.: | Step ID: | Step Description: |
| --- | --- | --- |
| 1 | pdp-cm-cfg-xx-yyyy | The connection should start after the contention has passed. The test environment should support only one active primary PDP context at particular moment. |
| Actions: | - Start first connection C1 for the SNAP #1 (to point to particular SNAP use TConnSnapPref parameter) - Start the connection C2 for the SNAP #2 using the Rconnection::Start() with TConnPrefList TConnAutoStartPref parameter. - Stop the connection C1 - Start the connection C3 for the SNAP #3 - Restart the connection C1using the Rconnection::Start() with TConnPrefList TConnAutoStartPref parameter. - Restart the connection C3 using the Rconnection::Start() with TConnPrefList TConnAutoStartPref parameter. - Start the connection C5 for the SNAP #5 using the Rconnection::Start() with TConnPrefList TConnAutoStartPref parameter. - Stop the connection C1 - Stop the connection C3 - Stop the connection C5 |  |
| Expected Result: | Step 1: The connection C1 started with no errors. Step 2: The connection C2 run asynchronous Start procedure with no errors Step 3: The connection C1 stopped with no errors, the connection C2 finishes the start procedure with no errors Step 4: The connection C3 takes context activation error code as result of the start procedure Step 5: The connection C1 is started immediately with no errors, the connection C2 is stopped because of contention Step 6: The connection C3 run asynchronous Start procedure with no errors Step 7: The connection C5 run asynchronous Start procedure with no errors Step 8: The connection C1 stops with no errors, the asynchronous procedure Start of the connection C3 finishes with no error Step 9: The connection C3 stops with no errors, the asynchronous procedure Start of the connection C5 finishes with no error Step 10: The connection C3 stops with no errors |  |

# Further Information

## People

| Role | Person / People |
| --- | --- |
| Contributor(s) |  |
| Reviewer(s) |  |

## References

| No. | Document Reference | Version | Description |
| --- | --- | --- | --- |
| [1] |  |  |  |
| [2] |  |  |  |

## Open Issues

The following issues need to be resolved before this document is completed;

- None

## Glossary

The following technical terms and abbreviations are used within this document.

| Term | Definition |
| --- | --- |
| API | Application Programmer Interface |
| SNAP | Service Network Access Point |

## Document History

| Date | Version | Status | Author | Description |
| --- | --- | --- | --- | --- |
| 07-22-2008 | 0.1 | Draft | Konstantin Filatov | First draft based upon SGL.PR0073.035 Test Specification Template Rev1.1. |
| 08-27-2008 | 0.2 | Draft | Alexander Soldatov | Draft |