USB Client Porting and Test How-To Document

| Security Classification: | Confidential |
| --- | --- |
| Document Reference: | SGL.GT0177.306 |
| Status: | Released |
| Version: | 3.5.1 |
| Last Revised: | 07/10/2008 |
| Team/Department : | Kernel & Hardware Services |
| Author(s): | Michael Petzold |
| Owner(s): | Michael Petzold |

| Approver(s): | <Names> | Distribution: | <Names> |
| --- | --- | --- | --- |

Contents

1	Introduction	3

1.1	Purpose and Scope	3

2	Overview	3

2.1	USB Client Base Driver Architecture	3

2.2	Endpoint Numbering Schemes	6

3	The USB PSL	7

3.1	Reading Data	8

3.2	Writing Data	10

3.3	Endpoint Zero Read Handling	10

3.4	Endpoint Zero Write Handling	10

3.5	Endpoint Zero Control Transfer Processing	11

3.6	Endpoint Resource Management	11

3.7	Using DMA	13

3.8	PSL -> PIL Upcalls	15

3.9	USB OTG	15

4	Porting the PSL	16

5	Implementation Considerations	17

5.1	Hardware dependencies	17

5.2	The ROM Image	18

6	Testing the Port	19

6.1	T_USBAPI	20

6.2	The USBIO Driver	21

6.3	USBRFLCT and T_USB	21

6.3.1	USBRFLCT	21

6.3.2	T_USB	23

6.4	T_USB_WIN and T_USB_DEVICE	25

6.5	USBIOAPP	25

6.6	USBCV (USB Command Verifier)	25

6.7	USBMSAPP (USB Mass Storage Application)	26

7	Further Information	27

7.1	People	27

7.2	References	27

7.3	Open Issues	27

7.4	Glossary	27

7.5	Document History	29

# Introduction

## Purpose and Scope

The USB Client Controller is a Physical Device Driver (PDD) which supports USB Device Controller (UDC) hardware within the Symbian OS.

This document describes how to port the lower portion, the Platform Specific Layer (PSL), of the USB Client PDD to a new hardware platform. This new hardware platform will usually be some UDC IP as part of the main ASSP, but it could also be an external UDC chip that is connected to the main CPU/MPU. The porting effort consists mainly of providing an implementation for a set of pure virtual -- and if required also some virtual -- C++ member functions of a generic USB device controller class.

A considerable part of this document is dedicated to a description of the available test tools and the actual test procedure for the USB driver implementation.

The reader is assumed to have read and understood the Universal Serial Bus Specification [1] for general information on USB, and the Symbian OS USB API document [2] for an introduction to the USB device driver stack architecture and implementation in Symbian OS.

# Overview

## USB Client Base Driver Architecture

Basically, Symbian's USB Base driver architecture looks like this:

Application            USB Class Driver

+-------------+  +------------------------------+

| +---------+ |  | +--------------+ +---------+ |

| |Interface| |  | |   Interface  | |Interface| |

| +----+----+ |  | +----+----+----+ +---------+ |

| | Ep | Ep | |  | | Ep | Ep | Ep | |   Ep    | |

| +----+----+ |  | +----+----+----+ +---------+ |

+-------------+  +------------------------------+

|                   |             |         user-side

----(1)------------------------------------------

|                   |             |         kernel-side

+---------+         +---------+   +---------+

| USB LDD |         | USB LDD |   | USB LDD |    usbc.ldd (n times)

+---------+         +---------+   +---------+         and/or

|                   |             |        usbcsc.ldd (n times)

+--(2)------------------------------------+

|              USB PDD (PIL)              |  \

|-------(3)-------------------------------|    > usbcc.dll (once)

|              USB PDD (PSL)              |  /

+-----------------------------------------+

|

+---------------+

|    Hardware   |       USB Device Controller (UDC)

~~~~~~~~~~~~~~~~~

There are three sets of interfaces in the USB driver stack (numbers refer to the diagram above):

(1)  the User-side/LDD interface for USB applications and class drivers (the "USB API"),

(2)  the LDD/PDD interface between USB LDD(s) and the USB PDD,

(3)  the PDD PIL/PSL interface between the generic PDD part and PDD Base ports.

Thus, the USB PSL forms the lower part of the USB physical device driver (PDD), the upper part being the Platform Independent Layer (PIL). All USB functionality that is not specific to a particular UDC is contained, as far as possible, in the PIL. This includes the standard ‘Chapter 9’ device state handling and request processing functionality. The PIL is part of Symbian OS.

The complete PDD (or USB Client Controller) is implemented as one large C++ class. The PIL provides a generic abstract base class DUsbClientController, which also contains a number of virtual and pure virtual functions. These virtual functions provide functionality which is not part of the USB specification and the way of affecting that functionality therefore normally differs from one UDC to the next. The functions, with other words, implement the direct UDC hardware manipulation: register access, endpoint FIFO loading/unloading, etc.

To create a fully functional and complete PDD, the PSL derives from the base class DUsbClientController, thereby creating a new platform-specific class D<SpecificPlatform>Usbcc which – at the least - implements the pure virtual functions. For the USB PDD implementation for we might have, for example, the following.

/e32/include/drivers/usbc.h defines the base class:

class DUsbClientController : public DBase

/**

This is the USB client software controller class.

*/

{

...

};

And the hardware-dependent part defines the derived class:

class TRefHardwareUsbcc : public DUsbClientController

/**

The hardware driver object proper.

*/

{

...

};

The base class DUsbClientController contains a number of virtual and pure virtual functions which need to be implemented by the PSL. To give an idea of what sort of functionality needs implementing, here is the complete list of functions as declared in /e32/include/drivers/usbc.h:

virtual TInt SignalRemoteWakeup() =0;

virtual void DumpRegisters() =0;

virtual TDfcQue* DfcQ(TInt aUnit) =0;

virtual TBool CurrentlyUsingHighSpeed();

virtual TInt SetDeviceAddress(TInt aAddress) =0;

virtual TInt ConfigureEndpoint(TInt aRealEndpoint, const TUsbcEndpointInfo& aEndpointInfo) =0;

virtual TInt DeConfigureEndpoint(TInt aRealEndpoint) =0;

virtual TInt AllocateEndpointResource(TInt aRealEndpoint, TUsbcEndpointResource aResource) =0;

virtual TInt DeAllocateEndpointResource(TInt aRealEndpoint, TUsbcEndpointResource aResource) =0;

virtual TBool QueryEndpointResource(TInt aRealEndpoint, TUsbcEndpointResource aResource) const =0;

virtual TInt OpenDmaChannel(TInt aRealEndpoint);

virtual void CloseDmaChannel(TInt aRealEndpoint);

virtual TInt SetupEndpointRead(TInt aRealEndpoint, TUsbcRequestCallback& aCallback) =0;

virtual TInt SetupEndpointWrite(TInt aRealEndpoint, TUsbcRequestCallback& aCallback) =0;

virtual TInt CancelEndpointRead(TInt aRealEndpoint) =0;

virtual TInt CancelEndpointWrite(TInt aRealEndpoint) =0;

virtual TInt SetupEndpointZeroRead() =0;

virtual TInt SetupEndpointZeroWrite(const TUint8* aBuffer, TInt aLength, TBool aZlpReqd=EFalse) =0;

virtual TInt SendEp0ZeroByteStatusPacket() =0;

virtual TInt StallEndpoint(TInt aRealEndpoint) =0;

virtual TInt ClearStallEndpoint(TInt aRealEndpoint) =0;

virtual TInt EndpointStallStatus(TInt aRealEndpoint) const =0;

virtual TInt EndpointErrorStatus(TInt aRealEndpoint) const =0;

virtual TInt ResetDataToggle(TInt aRealEndpoint) =0;

virtual TInt SynchFrameNumber() const =0;

virtual void SetSynchFrameNumber(TInt aFrameNumber) =0;

virtual TInt StartUdc() =0;

virtual TInt StopUdc() =0;

virtual TInt UdcConnect() =0;

virtual TInt UdcDisconnect() =0;

virtual TBool UsbConnectionStatus() const  =0;

virtual TBool UsbPowerStatus() const =0;

virtual TBool DeviceSelfPowered() const =0;

virtual const TUsbcEndpointCaps* DeviceEndpointCaps() const =0;

virtual TInt DeviceTotalEndpoints() const =0;

virtual TBool SoftConnectCaps() const =0;

virtual TBool DeviceStateChangeCaps() const =0;

virtual TBool CableDetectWithoutPowerCaps() const;

virtual TBool DeviceHighSpeedCaps() const;

virtual TBool DeviceResourceAllocV2Caps() const;

virtual TBool DeviceHnpHandledByHardwareCaps() const;

virtual void Suspend() =0;

virtual void Resume() =0;

virtual void Reset() =0;

virtual void Ep0ReadSetupPktProceed() =0;

virtual void Ep0ReceiveProceed() =0;

virtual TBool PowerDownWhenActive() const;

virtual TInt PowerDown();

virtual TInt PowerUp();

virtual TInt EnterTestMode(TInt aTestSelector);

virtual TInt OtgEnableUdc();

virtual TInt OtgDisableUdc();

## Endpoint Numbering Schemes

Two different numbering schemes are used within the USB implementation (endpoint numbers), and one external numbering scheme exists (USB addresses).

So-called virtual or logical endpoints are utilised by the LDD and presented to the application / class driver at the Symbian OS USB API. "Real" or physical endpoints are used at the interface between, as well as within, PIL and PSL. USB addresses for endpoints are the only form understood by the host side and are used for communication over the bus.

Virtual endpoints range in value from 1 to KMaxEndpointsPerClient (which is currently 5), and are local to a LDD which represents a USB interface. Through these virtual endpoints the application exchanges data over the USB. Logical endpoints are referred to by an application simply by number, using the LDD channel handle. Inside the PIL they are represented by the class TUsbcLogicalEndpoint.

For applications that implement a specific USB device class, and which therefore need to access endpoint zero, virtual endpoint 0 is available. This endpoint is special in a number of ways: it is never claimed with a SetupInterface() call because it doesn't belong to any particular interface, it is therefore always available, and it is (at the Symbian OS USB API) the only bi-directional endpoint. i.e. it can be both read from and written to. Endpoint 0 is not represented by a TUsbcLogicalEndpoint within the PIL.

Real endpoints are representations of endpoints physically present on a given device. They are instances of class TUsbcPhysicalEndpoint, and reside in an array DUsbClientController::iRealEndpoints which is owned and maintained by the PIL. Here, in order to facilitate easy array indexing, the numbering scheme uses "transformed" USB endpoint addresses. In that model, we represent each channel as a separate endpoint, RX (OUT) channel as iRealEndpoints[n] and TX (IN) channel as iRealEndpoints[n + 1]. If we look at USB endpoint addresses, we see:

7 6 5 4 3 2 1 0

---------------

D 0 0 0 P P P P

where D is 0 = RX / 1 = TX and PPPP represents the port number (0-15).

With this in mind, the array index is just:

7 6 5 4 3 2 1 0

---------------

0 0 0 P P P P D

or a "ROTATE-LEFT 1" instruction (which unfortunately doesn't exist on ARM).

Conversion between array index and USB address is done by two inline functions:

TUint DUsbClientController::EpIdx2Addr(TUint aRealEndpoint) const;

TUint DUsbClientController::EpAddr2Idx(TUint aAddress) const;

Array indexes are used, where a parameter is called "TInt aRealEndpoint".

Virtual endpoints are used, where a parameter is called "TInt aEndpointNum".

USB addresses are used, where a parameter is called "TInt aAddress".

Note:

The PSL may define a local, even more specific class for the endpoint representation, and arrange those endpoints in its own array. The PSL for the some reference hardware UDC does that – e.g. it defines a class struct TEndpoint which contains everything the PSL needs in order to manage an actual endpoint, and instantiates them within the derived controller class as

TEndpoint TReferenceHardwareUsbcc::iEndpoints[KUsbTotalEndpoints].

# The USB PSL

The PSL contains only that functionality which cannot be abstracted and made generic because different UDC designs are programmed and operate differently. Everything else that has to be common because it is defined in the USB specification is contained, as far as possible, in the PIL. For instance, the internal and/or external management of the endpoint FIFOs can be different: in some UDCs the endpoints have hardwired FIFOs of certain (albeit possibly configurable) sizes, whereas other UDCs have a certain total amount of FIFO RAM available that has to be shared between all endpoints in use in a given configuration. Or the way the chip is programmed can be different: some UDCs have a single register into which all commands and their parameters are written, whereas others are programmed via a number of special purpose control registers.

So, how exactly the UDC is being operated is hardware-specific and the implementer of the PSL is free to do whatever is necessary to enable the UDC. All this will be transparent to the PIL, which uses only the fixed set of virtual and pure virtual functions listed above to communicate with the PSL, and thus with the hardware.

In order for the PSL to be able to provide the services expected by the PIL, it is important for the person doing the implementation to understand the concept of data transfer in the USB client controller stack. Managing the transfer of data over endpoints is, together with the general administration of the UDC, the main task of the PSL. Data transfers generally are set up by the PIL by calling one of the following member functions of class DUsbClientController (these obviously already appeared in the list of virtual functions above):

TInt SetupEndpointRead(TInt aRealEndpoint, TUsbcRequestCallback& aCallback);

TInt SetupEndpointWrite(TInt aRealEndpoint, TUsbcRequestCallback& aCallback);

TInt SetupEndpointZeroRead();

TInt SetupEndpointZeroWrite(const TUint8* aBuffer, TInt aLength, TBool aZlpReqd);

TInt SendEp0ZeroByteStatusPacket();

As can be seen, the functions fall into two groups, depending on whether endpoint zero or a general endpoint is to be addressed; the handling of Ep0 and non-Ep0 endpoints is slightly different throughout the USB stack. Let’s first consider the general endpoints, which are used to actually transfer user data.

Both the read and write setup function for general endpoints take as an argument, in addition to the number of the endpoint to be used, a reference to a request callback structure. This structure is the key to the data transfer mechanism, and it is defined in /e32/include/drivers/usbc.h as follows:

/** A USB transfer request.

@publishedPartner

@released

*/

class TUsbcRequestCallback

{

public:

/** @internalTechnology */

TUsbcRequestCallback(const DBase* aOwner, TInt aEndpointNum, TDfcFn aDfcFunc,

TUsbcEndpoint* aEndpoint, TDfcQue* aDfcQ, TInt aPriority);

/** @internalTechnology	*/

~TUsbcRequestCallback();

/** @internalTechnology	*/

inline void SetRxBufferInfo(TUint8* aBufferStart, TPhysAddr aBufferAddr,

TUsbcPacketArray* aPacketIndex,

TUsbcPacketArray* aPacketSize, TInt aLength);

/** @internalTechnology	*/

inline void SetTxBufferInfo(TUint8* aBufferStart, TPhysAddr aBufferAddr,

TInt aLength);

/** @internalTechnology	*/

inline void SetTransferDirection(TTransferDirection aTransferDir);

/** @internalTechnology	*/

inline const DBase* Owner() const;

/** @internalTechnology	*/

inline TInt DoCallback();

/** @internalTechnology	*/

inline void Cancel();

public:

/** Used by the PIL to queue callback objects into a TSglQue.

@internalTechnology

*/

TSglQueLink iLink;

public:

/** The endpoint number. */

const TInt iEndpointNum;

/** Indicates the LDD client for this transfer. */

const DBase* const iOwner;

/** DFC, used by PIL to call back the LDD when transfer completes to the LDD. */

TDfc iDfc;

/** Direction of transfer request. */

TTransferDirection iTransferDir;

/** Start address of this buffer. */

TUint8* iBufferStart;

/** Physical address of buffer start (used for DMA). */

TPhysAddr iBufferAddr;

/** Array of pointers into iBufferStart (actually TUsbcPacketArray (*)[]). */

TUsbcPacketArray* iPacketIndex;

/** Array of packet sizes (actually TUsbcPacketArray (*)[]). */

TUsbcPacketArray* iPacketSize;

/** Length in bytes of buffer (iBufferStart). */

TInt iLength;

/** For IN transfers, if a zlp might be required at the end of this transfer. */

TBool iZlpReqd;

/** Number of bytes transmitted; changed by the PSL. */

TUint iTxBytes;

/** Number of packets received (if it is a read); changed by the PSL. */

TUint iRxPackets;

/** The error code upon completion of the request; changed by the PSL. */

TInt iError;

};

## Reading Data

The PSL is, via the TUsbcRequestCallback structure, supplied with a large buffer (iBufferStart) of size iLength for the data to be read. For Bulk reads, it is intended that this buffer is used to catch a burst of data transmitted from the host, rather than for just a single packet. For other transfer types (Control, Interrupt, and Isochronous) normally single packets only are returned to the PIL/LDD. The amount of data returned is controlled (and limited) by the LDD through iLength.

Two arrays are also supplied: the iPacketIndex array contains the offset(s) into the data buffer of the single packet or burst data. The iPacketSize array contains the size(s) of the packet or burst. These arrays are logically linked, they are both the same size and can hold information for 2 entries each. (The reason for using an array for managing only two values is historical, and the functionality is now kept to maintain compatibility with existing PSL implementations.)

Therefore, received single packets have to be coalesced into one 'superpacket' in the read buffer. It is assumed that coalesced packets consist of maximum packet sized packets, possibly terminated by a short packet. A ZLP must appear separately; this would be the optional second packet in the respective array.

For example, for a Bulk endpoint with a maximum packet size of 64 bytes:

- If 10x 64 bytes packets and one 10 bytes packet arrive, then these are to be marked as a single large 650 byte packet.

- If 10x 64 bytes packets and one zero length packet arrive, then these should be entered as 2 packets in the arrays: one of size 640 bytes and one of size zero.

The general aim when servicing a Bulk read request from the PIL is to capture as much data as possible whilst not holding onto data too long before returning the data to the LDD and receiving another buffer from it. There is considerable flexibility in how this is achieved and the design does not mandate any particular method; it also depends to a certain extent on the UDC hardware being used.

A recommended design is described here.

A timer is used to see if data has been received in the n milliseconds since the last data was received. If data has been received, then the timer is restarted. If data has not been received then the timer is not restarted and the buffer is completed to the PIL using EndpointRequestComplete().

The following should be borne in mind:

- In the ISR a flag is used to indicate that data has been received.

- Do not restart the timer in the ISR. Let the timer expire and test for received data in the timer callback.

- Typical values for the timer include 1—5 ms.

- Each OUT endpoint will require a separate timer.

- Do not complete to the PIL if no data at all has been received. Only start the timer once the first packet/transfer has been received.

- Keep work to the bare minimum in the ISR. After draining the FIFO, update the iPacketSize and iPacketIndex arrays (if necessary) and recalculate the buffer address ready for the next packet, taking into account any required alignment restrictions.

The PSL completes a non-Ep0 read request by calling the PIL function EndpointRequestComplete(). This function takes as its sole argument a pointer to the updated request callback structure that was initially passed to the PSL for that read. Fields that need to be updated, in addition to the packet arrays iPacketIndex and iPacketSize, are iRxPackets (possible values: 1 or 2) and iError.

Summary of Completion Criteria for Bulk Endpoints

The PSL shall complete the read by calling EndpointRequestComplete(), when any of the following conditions are met:

- The requested number of bytes have been received.

- A short packet, including a ZLP, is received. In the case of a ZLP being received, it must be represented separately in the iPacketIndex and iPacketSize arrays.

- The number of bytes in the current packet (still in the FIFO) would cause the total number of bytes to exceed the read buffer length iLength.

- The timer has expired, data is available, but no further packets have been received.

## Writing Data

A data write is simpler to handle than a Read, because with a write the PSL knows beforehand how much data is involved in the transfer. As in the Read case, the PSL is supplied, via the request callback argument of SetupEndpointWrite(), with a pointer to a buffer, which in this case is constant and contains the data to be sent in one consecutive piece of memory. All that needs to be done for the PSL is setting up the transfer (via DMA or conventionally interrupt driven) and waiting for the host to collect, by sending an IN token, the data from the respective endpoint’s primed FIFO. This goes on until all data from the buffer has been transmitted to the host.

If the ZLP request flag iZlpReqd in the TUsbcRequestCallback structure is set (= ETrue), then the PSL will have to determine, after all data of the write request has been sent, whether to send a ZLP or not. This decision will be based on the size of the last packet sent and the current max packet size of the endpoint. In other words: a ZLP should be sent at the end of a write request if

- the ZLP flag is set, and

- the last packet of the write request was not a short packet (i.e. was a max-packet-sized packet)

As with a read, the PSL completes a non-Ep0 write request by calling the PIL function EndpointRequestComplete(). Fields in the request callback structure that need to be updated by the PSL for a write are iTxBytes and iError.

## Endpoint Zero Read Handling

The handling of endpoint zero read requests is in general similar to that of general endpoints, however there are also some differences. The request is set up by the PIL by calling the function SetupEndpointZeroRead() instead of SetupEndpointRead(). SetupEndpointZeroRead() doesn’t take any parameters because all the information that is transmitted in the arguments of SetupEndpointRead() is here, in the endpoint zero case, either implicitly known or just not relevant. The endpoint number we already know: it’s zero. We also don’t need a TUsbcRequestCallback structure, because we complete to the PIL after every received packet. Therefore all that is important to know for the PSL is where to place the received data, and this location is fixed: it’s iEp0_RxBuf and its size is KUsbcBufSzControl. An endpoint zero read request is completed by calling Ep0RequestComplete(). Since we don’t use a request callback structure with Ep0, the endpoint number (0 or KEp0_Out  for a read), the number of bytes processed (here: received) and the resultant error code for this request are passed directly as arguments to this function.

## Endpoint Zero Write Handling

The handling of endpoint zero write requests is again done by a separate function, SetupEndpointZeroWrite(). Different from the Ep0 read function however, this one does take arguments. Since the location (which could be anywhere) and the size of the data to be sent are not known to the PSL prior to the write request, these values have to be passed as parameters to the write function. There is also a third parameter, aZlpReqd, which specifies whether or not a zero length packet (ZLP) should be sent directly after the data has been transmitted (if required).

There is one additional Ep0 write function, SendEp0ZeroByteStatusPacket(), which is used for sending a ZLP on its own. The reason for having a separate function for this purpose is that the mechanisms of getting the UDC to transmit a ZLP on its own and at the end of a data transmission can be different.

An endpoint zero write request is completed by calling Ep0RequestComplete(). Again, the endpoint number (1 or KEp0_In  for a write), the number of bytes processed (here: sent) and the resultant error code for this request are passed directly as arguments to the function.

## Endpoint Zero Control Transfer Processing

The PSL will in general have to make sure that it is always acting in accordance with the three-stage protocol as described in [1] 8.5.3 "Control Transfers" when serving endpoint zero.

For this purpose it may be useful (and indeed necessary) to introduce an Ep0 state variable which keeps track of the current stage, records whether or not there is a Data stage, and remembers the direction of the Status stage. Reference hardware might use this for the (hardware-dependent) values of enum TEp0State.

The processing of the Setup and (optional) Data stages consists mainly of sequences of reading and writing packets from and to the endpoint as described in the previous two sections. Basically, all received Setup and Data packets are to be completed to the PIL and nothing is to be transmitted that didn't come from the PIL.

The processing of the Status stage is slightly different though:

ZLPs (status packets) are only sent to the host when the PIL calls SendEp0ZeroByteStatusPacket(). The successful (or otherwise) completion of the Status stage (i.e. the ACK of the host in response to the ZLP) is not reported to the PIL.

ZLPs received from the host during the Status stage are ACK'ed by the PSL in the normal way, but then ignored (or 'swallowed') and not completed to the PIL.

## Endpoint Resource Management

In July 2008, a new method for requesting endpoint resources (DMA, Double-buffering) was introduced in Symbian OS v9.5. Instead of having synchronous resource allocation and deallocation functions, with the new scheme the required endpoint resources are indicated by the user (app or class driver) only once at interface setup time. The main reason for introducing a new resource allocation scheme was the requirement to provide a mechanism which enabled the PSL to manage DMA resources more efficiently than is possible with the existing (now the ‘old’) scheme.

### The Old Resource Allocation Scheme

The old scheme uses the synchronous user-side USB LDD APIs

TInt RDevUsbcClient::AllocateEndpointResource(TInt aEndpoint,

TUsbcEndpointResource aResource);

TInt RDevUsbcClient::DeAllocateEndpointResource(TInt aEndpoint,

TUsbcEndpointResource aResource);

TBool RDevUsbcClient::QueryEndpointResourceUse(TInt aEndpoint,

TUsbcEndpointResource aResource);

which translate at the PDD PIL/PSL boundary (after endpoint number conversion) into these pure virtual functions:

TInt DUsbClientController::AllocateEndpointResource(TInt aRealEndpoint,

TUsbcEndpointResource aResource);

TInt DUsbClientController::DeAllocateEndpointResource(TInt aRealEndpoint,

TUsbcEndpointResource aResource);

TBool DUsbClientController::QueryEndpointResource(TInt aRealEndpoint,

TUsbcEndpointResource aResource) const;

The PSL implementation will try to allocate (or free) a requested resource immediately and report the outcome via the respective function’s return value.

These APIs, except for the QueryEndpointResource calls, were classified as deprecated in Symbian OS v9.5 in July 2008. This means that while the user-side APIs are still present and supported, they shouldn’t be used for new application development work as they are likely to be removed at some point in the future. Of course, since the DUsbClientController methods are pure virtual, they will have to be provided by PSL implementations as long as they are there.

### The New Resource Request Scheme

The new endpoint resource request scheme was introduced at the same time. It works differently (unsurprisingly) and in order for an application to be able to use it, the PSL has to explicitly support it as well. The PSL indicates if this is the case by overriding the default implementation of base class virtual function

TBool DUsbClientController::DeviceResourceAllocV2Caps() const;

(which returns EFalse) and make it return ETrue instead.

As was mentioned above, with the new scheme the required endpoint resources are selected by the user at USB interface setup time. The resource requests are specified in the structure TUsbcEndpointInfo via flags in the member variable iFeatureWord1:

/** The endpoint resource allocation flags;

used by TUsbcEndpointInfo::iFeatureWord1.

@see TUsbcEndpointInfo

*/

const TUint KUsbcEndpointInfoFeatureWord1_DMA             = 0x00000001;

const TUint KUsbcEndpointInfoFeatureWord1_DoubleBuffering = 0x00000002;

This structure is passed as part of structure TUsbcInterfaceInfo to the LDD and the PIL at interface (and thus logical endpoint) creation time in the user-side call RDevUsbcClient::SetInterface(). The PIL will not try to actually allocate any of the requested resources at that point, so the successful outcome of the call does not depend on the availability or otherwise of those.

Instead, the resource request information is left where it is in the PIL (which is in the endpoint info structure) and is passed to the PSL as a parameter of pure virtual function DUsbClientController::ConfigureEndpoint() which is called by the PIL in function DUsbClientController::InterfaceSetup()(and only there).

InterfaceSetup() gets called in response to

- the host selecting a new configuration (with default alternate interface settings) by sending a  SET_CONFIGURATION request,

- the host selecting a single alternate interface setting by sending a SET_INTERFACE request.

In the ConfigureEndpoint() call the PSL has all the information it needs to allocate the requested endpoint resources.

(For the user this means there is no longer a direct and immediate feedback as to whether resources were successfully allocated or not, but they can find out by calling RDevUsbcClient::QueryEndpointResourceUse()after a Configuration or Interface Alternate Setting change.)

The PSL may (and probably should) deallocate the resources for an endpoint within the respective DUsbClientController::DeConfigureEndpoint() call which is issued by the PIL in function DUsbClientController::InterfaceSetTeardown().

InterfaceSetTeardown() gets called in response to

- the host selecting a different configuration (incl. 0) than the current one by sending a  SET_CONFIGURATION request,

- the host selecting a single different alternate interface setting than the current one by sending a SET_INTERFACE request,

- the user calling RDevUsbcClient::ReleaseInterface() or  unloading the USB Client LDD.

Just as there exists no longer an explicit resource allocation request call, there is no explicit deallocation request call either. The resource requests exist over the lifetime of the respective interface (setting). Whenever a particular endpoint becomes active as part of an interface setting, and thus gets configured by the PIL, the indicated resources will be allocated by the PSL. Likewise, whenever that endpoint becomes inactive and gets deconfigured, the PSL will free the resources again.

### Compatibility Considerations

A PSL that chooses to support the new resource scheme will have to support the old scheme as well (at least to some extent) as long as the pure virtual functions AllocateEndpointResource() and DeAllocateEndpointResource() are still part of the controller base class.

## Using DMA

When planning to use DMA for transferring data from and to the endpoints’ FIFOs a few things have to be considered.

1)  The cached information for the data buffers needs to be flushed before a DMA write (i.e. transfer memory -> FIFO) and before and after a DMA read (i.e. transfer FIFO -> memory). The kernel provides three functions for that purpose (declared in /e32/include/kernel/cache.h -- #include <cache.h>):

void Cache::SyncMemoryBeforeDmaWrite(TLinAddr aBase, TUint aSize);

void Cache::SyncMemoryBeforeDmaRead(TLinAddr aBase, TUint aSize);

void Cache::SyncMemoryAfterDmaRead(TLinAddr aBase, TUint aSize);

The following three code fragments from a reference USB PSL illustrate when and how these functions can be used.

void TReferenceBoardUsbcc::BulkTransmit(TInt aEndpoint)

{

const TInt idx = aEndpoint * 2 + 1;		// get endpoint array index

const TEndpoint* const ep = &iEndpoints[idx];	// get pointer to endpoint

const TUint8* const buf = ep->iTxBuf;		// get pointer to data buffer

const TInt len = ep->iLength;		// no. of bytes to be sent in total

// Prepare DMA descriptor(s) (for Scatter/Gather controller):

...

// Then, flush data cache and queue DMA transfer request

// (here: prepare for DMA write):

Cache::SyncMemoryBeforeDmaWrite(reinterpret_cast<TLinAddr>(buf), len);

DDmaRequest* const req = ep->iDmaRequest;

req->Queue();

}

void TReferenceBoardUsbcc::BulkReadSetupDma(TInt aEndpoint)

{

const TInt idx = aEndpoint * 2;			// get endpoint array index

const TEndpoint* const ep = &iEndpoints[idx];	// get pointer to endpoint

const TUint8* const buf = ep->iRxBuf;		// get pointer to data buffer

const TInt len = ep->iLength;		// no. of bytes to be read in total

// Prepare DMA descriptor(s) (for Scatter/Gather controller):

...

// Then, flush data cache and queue DMA transfer request

// (here: prepare for DMA read):

Cache::SyncMemoryBeforeDmaRead(reinterpret_cast<TLinAddr>(buf), len);

DDmaRequest* const req = ep->iDmaRequest;

req->Queue();

}

void TReferenceBoardUsbcc::BulkReceiveDmaDfc(DDmaRequest::TResult aResult, TAny* aArg)

//

// (This function is static.)

//

{

TEndpoint* const ep = static_cast<TEndpoint*>(aArg);

const TInt ep_num = ep->iEndpointNumber;

// Check 'aResult':

...

// Update some request values:

...

// (Through the cast to a refboard pointer we are permitted access to

//  the protected base class function:)

TReferenceBoardUsbcc* const refboard =

static_cast<TReferenceBoardUsbcc*>(DUsbClientController::UsbClientController);

// Then, flush data cache and complete read request to the LDD:

Cache::SyncMemoryAfterDmaRead(reinterpret_cast<TLinAddr>(ep->iRxBuf),

ep->iReceived);

refboard->RxComplete(ep);

}

2)  Whereas the implementation of DMA mode for IN transfers normally should be relatively straightforward, with OUT transfers (DMA reads) there could be some complications, depending on the DMA controller and the UDC and the way the two are connected.

One issue concerns the fact that, if we set up a DMA read for, say, 4KB and this request returns after a short packet, we have to be able to tell how much data has been received and is now in our buffer. That is, the DMA controller has to allow us somehow to find out how many bytes have been received, for otherwise we cannot complete the read request to the LDD.

Here is one theoretical solution for a Scatter/Gather controller that doesn’t provide information about the number of bytes transferred directly (this proposal has not actually been tested!). Instead of using one large DMA descriptor for 4KB, we could set up a chain of descriptors for max-packet-size bytes each. Then, when the DMA completes, it will either be for the whole transfer, in which case we know the number of bytes received, or it will be due to a short packet. In the latter case we could try and find out which descriptor was being served at the time of the short packet. The number of bytes received is then: ‘number of completed descriptors * max-packet-size + number of bytes in short packet’.

Another potential problem could be posed by the restartable OUT endpoint timer, which was described earlier in this chapter under ‘Reading Data’. Here, the situation in the timer callback DFC, when the LDD read request is about to be completed, has to be handled with care in order not to lose data. If at that point the pending DMA request has already started transferring data from the endpoint FIFO into our data buffer, then we cannot complete the LDD request any more – the data would be lost because the LDD wouldn’t know about it (the variables in the request structure don’t take into account the new data). And the removal of the buffer into which, at that point, data is being DMA’ed clearly must not be allowed to happen anyway.

A possible solution would be as follows (again, this is just an idea and therefore untested). In the timer DFC, before we complete to the LDD, we cancel the pending DMA request. If there was then already a DMA transfer going on, then this will somehow complete (on some hardware the current burst will be finished before the DMA controller stops), and the DMA complete DFC will be queued. What we need to find out in that situation is whether or not that DFC is pending. If it is not, then there was no DMA transfer ongoing, and we can just complete our read request to the LDD. If however it is, then we have to abandon at this point the plan to complete and just return from the timer callback (without having done any damage). In that case, the DMA complete DFC will be run next, the LDD read request structure will be updated, the RX timer will be set again, and we can proceed just normally.

## PSL -> PIL Upcalls

The PIL class DUsbClientController provides a number of functions that are meant to (also) be called by the PSL. These are currently:

TInt DeviceEventNotification(TUsbcDeviceEvent aEvent);

void EndpointRequestComplete(TUsbcRequestCallback* aCallback);

TInt Ep0RequestComplete(TInt aRealEndpoint, TInt aCount, TInt aError);

void MoveToAddressState();

void SetCurrent(TInt aCurrent);

TUsbcEp0State EnquireEp0NextState(const TUint8* aSetupBuf) const;

TInt ProcessSetConfiguration(const TUsbcSetup& aPacket);

void HandleHnpRequest(TInt aHnpState);

While some of them will always and necessarily have to be called by the PSL (like EndpointRequestComplete() in order to signal a data transfer request completion), others are there just as a service (for instance EnquireEp0NextState()) or their calling may only be required under certain circumstances (for example ProcessSetConfiguration() in case the UDC ‘swallows’ and thus doesn’t pass on SET_CONFIGURATION requests).

For details on the use of those functions see the in-source comments preceding their implementations in ps_usbc.cpp and chapter9.cpp, respectively.

All the above functions can be called either in ISR context or in Thread context (via a DFC). It is however probably safer to use only one context type (ISR or Thread) for all  PIL upcalls in a given PSL implementation. For example, a FS USB driver might always call the PIL functions in ISR context, while the an HS USB driver might always call them in Thread context.

## USB OTG

When implementing a USB Client driver that is to work alongside the Symbian USB Host/OTG stack, two additional virtual functions will probably need to be filled in with proper functionality by the PSL:

virtual TInt OtgEnableUdc();

virtual TInt OtgDisableUdc();

These two functions are called by the PIL after StartUdc() and before StopUdc(), respectively. As to what exactly needs to be done in each function is probably largely hardware-specific. One thing the functions should not do though is manipulate the D+ pull-up resistor via the transceiver - this would also in an OTG setup only be done through the existing PSL functions UdcConnect() and UdcDisconnect() (if they are called at all in that case).

Both the normal and the OTG version of the USB Client PDD are compiled from the same source, but with the macro USB_OTG_CLIENT defined in the case of the OTG version.

The presence or otherwise of the USB_OTG_CLIENT macro is used in the PIL also to affect one major functional difference in the case of the OTG-capable version of the driver: it starts off disabled. The Client stack needs to be explicitly enabled by the OTG stack before it can become operational. This is because in an OTG setup it is not clear at boot time whether the phone will start out as an A- or a B-device.

Another important difference is the way the bus connection / disconnection via the transceiver is handled, with the OTG stack (and not the Client stack) exclusively in charge of this in the OTG case.

# Porting the PSL

As has already been mentioned, the actual porting task consists mainly of providing implementations for all of the pure virtual functions and any number of the virtual functions of the UDC base class DUsbClientController. Here follows a proposed step-by-step guide of what needs to be done.

- In the ASSP directory that should hold the PSL files (normally the UDC is part of the ASSP, for an external UDC one would probably use the Variant directory instead), create two files for the PSL implementation:

/refboardname/pa_usbc.cpp

/ refboardname /pa_usbc.h

- Implement the PSL within these two files. (Of course the PSL can also be split over more files if that's more convenient.) UDC register definitions and register bit definitions will probably go into <ASSP>.h, if such a file exists.

There are in-source comments in the base class header file /e32/include/drivers/usbc.h, which explain what each of the virtual functions is supposed to do.

When putting debug printfs in the code, use the KTRACE macro with either the KPANIC flag for error messages or the KUSBPSL flag for general informational output:

__KTRACE_OPT(KPANIC, Kern::Printf("Error: USB Controller Not Present"));

__KTRACE_OPT(KUSBPSL, Kern::Printf("Received new Ep0 Setup packet"));

Do not use the KUSB flag, as that is used within the PDD PIL and the USB LDD.

- Add to the same directory another file:

/refboardname/usbcc.mmp

This is the MMP file for the PDD; it lists – among other things - all the source files that are to be compiled for the PDD (not just the PSL!) and the libraries against which the object files should eventually be linked.

- Add the name of the .mmp file (without the file extension) to the <ASSP>.inf in the same directory. This tells abld about the existence of the .mmp file.

- In some hardware configurations, the PSL is also dependent on certain Variant-specific functionality; for more information regarding this see section 5.1”Hardware dependencies”.

- Finally, the USB drivers need to be added to the respective .iby and .oby files, for more information on this see section 5.2 “The ROM Image”.

# Implementation Considerations

## Hardware dependencies

There exist hardware configurations where some PSL functionality is dependent not only on the UDC (ASSP) alone, but also on the Variant (i.e. the device or development board).

The USB cable connection/disconnection notification mechanism, for instance, is not defined (not even mentioned) in the USB specification, Therefore, some UDCs have ASSP-internal means for cable connection status detection, but some have not. Here one can often find some Variant specific hardware mechanism, which normally requires also some software support in the Variant layer.

With the some hardware cable status detection might be done by some additional hardware on the reference board (and not by the processor). To utilize this functionality and make it available to the USB PSL, additional code must be added to ASSP Variant library class definition to add a number of pure virtual functions for enquiry of the cable status detection, which will then be implemented by the Variant..

## The ROM Image

The two USB drivers (LDD EUSBC.LDD and PDD USBCC.DLL), as well as any other required components (for example a USB transceiver DLL) need to be added to the ROM image. The following lines are taken, as an example, from the Lubbock port’s /lubbock/rom/kernel.iby file:

// USB LDD

// (The target file name has to be lower case, for otherwise it collides with the macro 'EUSBC'!)

DEVICE[VARID]=	/EPOC32/RELEASE/##KMAIN##/##BUILD##/USBC.LDD \SYS\BIN\eusbc.ldd

// USB PDD

EXTENSION[VARID]=/EPOC32/RELEASE/##KMAIN##/##BUILD##/_##VARIANT##_USBCC.DLL \SYS\BIN\USBCC.DLL

From a reference hardware specific .iby file:

// USB drivers

#ifdef SYMBIAN_EXCLUDE_USB

REM Feature USB is not included in this ROM

#else

extension[VARID]=KERNEL_DIR/DEBUG_DIR/_lubbock_usbcc.dll  \sys\bin\usbcc.dll

// (The target file name has to be lower case, for otherwise it collides with the macro 'EUSBC'!)

device[VARID]=KERNEL_DIR/DEBUG_DIR/usbc.ldd                  \sys\bin\eusbc.ldd

#endif // SYMBIAN_EXCLUDE_USB

Which of the two .iby files is being used depends on the particular development environment and target. A text-shell ROM will normally use the kernel.iby file, whereas a full ROM often uses the base_reference hardware.iby file.

Furthermore, the command line test applications need to be added; this could be done in a special-purpose .oby file which is used directly to create a text shell test ROM image:

FILE=/EPOC32/RELEASE/ARMV5/UREL/T_USB.EXE     SYS\BIN\T_USB.EXE

FILE=/EPOC32/RELEASE/ARMV5/UREL/T_USBAPI.EXE	 SYS\BIN\T_USBAPI.EXE

(But note that both test applications are also pulled in by default by the standard e32tests.oby file.)

And finally, the macro EUSBC needs to be defined for the base port in order to get the user-mode components (like USBMan) pulled in for a full ROM image. This is often done in the reference hardware .oby file:

REM Define whether or not to include USB Device Controller support:

#define EUSBC

# Testing the Port

There are a number of steps, or stages, to testing a port. A USB analyser, like the ones made by LeCroy (formerly CATC), is invaluable for this work -- in fact, it should be almost impossible to omit this piece of equipment and be sure that the port is successful. For testing the ported driver one will need:

- A Symbian OS text shell ROM image for the device that also contains the USB drivers (PDD and LDD) and the unit test applications T_USB and T_USBAPI. Depending on the processing power of the main CPU, it may be necessary to use UREL versions of the driver binaries in order not to exceed the time-out limits of the USB protocol (which could prevent even a successful device enumeration).

- A PC running either MS Windows [98SE|ME|2000|XP] to run the Symbian host-side test program USBRFLCT, or MS Windows XP to additionally be able to run the USBCV compliance test program.

- A USB driver floppy disk (or other location accessible for the test PC) containing the .INF files that match the device to be tested and the driver binary files one of which will be installed for the device. (All these items are included in the binary test program distribution; see 6.3.1 “USBRFLCT”).

Here are the proposed steps for testing a port.

- Run T_USBAPI on the unconnected USB device and get it to pass.

- Use a USB cable to connect the device under test to a PC and start T_USB on the device (use T_USB menu options '0' '1' '1' '1'). Now you will see if the device successfully delivers all the information (descriptors) that the host asks for. If so, a dialogue box should pop up on the PC, saying something like “New hardware found” and offering certain options. Point the OS to the location of the .INF files and the driver files (the USB driver disk) and have it pick and install the correct driver. After that, one should be able to see the new USB device in the Windows Device Manager under category ‘Symbian OS controlled USB devices’ as ‘USBRFLCT Device’. If things don’t work as expected at this early stage, the USB analyser will prove invaluable.

- Try and run USBRFLCT and T_USB together. First start T_USB on the device, choose the four numbered program options (but this time the first menu option mustn't be '0'), and then start USBRFLCT on the PC. The program pair should run indefinitely, in any mode. If the program halts after a while then this will most likely indicate either a hardware problem associated with the UDC or an incorrect handling of the UDC in the PSL. If the halt occurs after a long running time then this could also point to a resource problem, such as a memory leak. This test will verify that at least one bulk IN endpoint and one bulk OUT endpoint are working correctly and that data can be transferred over the USB link.

- Run the USB-IF’s USBCV compliance test program suite and go through the “Chapter 9 Tests”. One will also need to start T_USB on the device prior to that, as USBCV needs a successfully enumerated device as a target (choose '0' as the first T_USB option). These tests should all pass. Running USBCV is important because it verifies that Ep0 is behaving correctly and that the device is compliant to the USB Specification 2.0.

It now follows a more thorough description of the test programs just mentioned (and also of some which haven't been mentioned).

## T_USBAPI

T_USBAPI is a device-side only, stand-alone test program. It is used to verify the proper working of some basic user-side USB API functionality. For T_USBAPI to be run, the device doesn’t need to be USB-connected to a PC, but it can, and if so then a few additional tests are performed. Tests run by T_USBAPI include

- loading the USB LDD and opening a channel,

- searching for suitable endpoints and setting up an interface,

- searching for suitable endpoints and creating an 'alternate setting' for that interface (if supported),

- querying and allocating endpoint resources (DMA and double buffering),

- powering up the UDC and connecting it to the bus,

- manipulating (modifying) USB descriptors (Device, Device_Qualifier, Configuration, Other_Speed_Configuration, Interface, Endpoint, String),

- testing the USB OTG API extensions,

- requesting and releasing ‘Device Control’,

- some static state notification tests (alternate device status, endpoint status),

- setting and clearing endpoint Halt condition (where supported in unconfigured state),

- closing the channel and freeing the LDD.

Most of these tests are there to verify the functioning of the platform-independent code (PIL + LDD). However some of this code relies on platform-specific (PSL-provided) functionality, such as the stall state and the max packet size of endpoints. Therefore an at least partially working PSL is required for T_USBAPI to pass.

When called normally, T_USBAPI runs once and then exits. It can however also be run endlessly in a loop (provided it’s not failing, of course), by appending the parameter ‘soak’ on the command line. Once the program is running in endless-mode it can be stopped at any time by pressing the ‘Esc’ key. In that case the program will first finish the current set of tests and then exit. This loop mode can be useful for the detection of memory leaks in the USB stack, although, as indicated, the main functionality thus exercised lies in the PIL and LDD, not in the PSL.

Since T_USBAPI is a standalone program, it can safely be run on a UDEB image. Error messages and warnings specific to the USB components can be chosen to be displayed by setting the respective kernel debug flags KPANIC, KUSBPSL, and KUSB. The following E32 text shell commands set these (first KPANIC, then both KUSBPSL and KUSB):

C:\>trace 80000000

C:\>trace 3 1

"trace 3 1" means "set (bit 1 | bit 0) at index 1" (KUSBPSL = 33, KUSB = 32).

## The USBIO Driver

The host (PC) side test program that is described below, USBRFLCT, uses the services of a certain USB client driver, USBIO.SYS. Once properly installed on the PC, this driver will be loaded by the OS – as part of the enumeration -- when the USB device that is to be tested gets connected to the PC.

The generic USB device driver USBIO is a product developed and marketed by German company Thesycon [3]. The driver provides Win32 applications with direct access to USB devices which are otherwise only accessible from kernel mode. It can be used with any kind of USB device, enabling application developers to control devices without having to develop a kernel-mode WDM driver.

## USBRFLCT and T_USB

These are the host- and device-side parts, respectively, of a reflector-like USB unit test program. They can only be used together*.

Note:

Since this program suite is subject to continual development and improvement, in order for them to work properly together, both parts always should come from the same Symbian OS delivery. To ensure proper interworking, the programs exchange their version numbers after startup, and either program will quit if the other one doesn't provide a certain, minimally required version number.

(* Not quite true: T_USB can also be used to create a simple USB device for USBCV to exercise.)

### USBRFLCT

USBRFLCT is the host-side part. Its source code can be found under /e32test/win32/usbrflct/. The directory /e32test/win32/usbrflct_distribution/ contains a binary distribution of the program, including the required USBRFLCT.SYS driver and a USBRFLCT.INF file for the installation of the driver on the PC.

*--------------------------------------------------

* USBRFLCT v1.6.0 (for use with USBRFLCT.SYS v2.41)

* USB Reflector Test Program / T_USB Host-side Part

*   Copyright (c) 2001-2007 Symbian Software Ltd.

*--------------------------------------------------

* Syntax: usbrflct [options]

* Options:

* /[r|t]    receive|transmit only; default is to loop

* /z		zero length packet at end of a transmission

*		(only if last packet is full length)

* /l        loop mode with stats printed for every iteration

* /v        verbose driver & program output

* /[h|?]    displays this help text

There are three modes of operation available: Loop, Receive-only, and Transmit-only. Loop mode is the default, the other two modes are selected via the /r and /t command line options, respectively.

In Loop mode data chunks of increasing size (one byte at a time), starting with size 0 and up to a certain maximum size (selected on the device side in T_USB, see below), are read by the host from the USB device (IN transactions) and are immediately sent back to the device (OUT transactions). After the maximum size has been reached, the chunk size starts anew with its starting value. Prior to each data chunk a “preamble” packet is sent by the device, announcing the size of the chunk that will follow. This kind of protocol is necessary, because with the driver used on the host side every read request has to specify exactly the number of bytes intended to be read (= sent by the device).

The program displays in regular intervals – every 1024th loop -- some statistics about the ongoing test run, for instance the total number of bytes transferred and the current average data throughput in bytes/sec. Specifying the option /l on the command line lets the program update that information after every iteration (loop). Since this slows the program down, it is not the default

Receive-only mode uses only at the beginning one single preamble packet from the device carrying the transfer size, and henceforth chunks of that size are continuously read by the host-side program. This mode can be used to determine the raw transfer performance of the USB connection in the device upstream (IN) direction.

Transmit-only mode also uses a single preamble packet at the beginning, and then continuously sends data chunks of constant size to the device. This mode can be used to determine the raw transfer performance of the USB connection in the device downstream (OUT) direction.

With the /z command line option transmission from the host includes a final zero length packet for any transmission that would otherwise have a full length packet as the final packet, i.e. for any transmission with a length that is a multiple of the maximum packet size for that endpoint. For transmission lengths that are not a multiple of maximum packet size then the final packet is a short packet and this command line option has no effect.

With the /v command line option verbose output can be activated. The output, most importantly the error messages, can be used to track down problems when trying to get USBRFLCT and T_USB to work together, either for the first time after the PSL port, or after installation changes on the PC. The error codes of the USBRFLCT.SYS driver can be found explained in the USBIO Reference Manual which is part of the USBIO development kit installation; alternatively a copy is available on Thesycon’s website at

http://www.thesycon.de/usbio/usbioman.pdf.

Due to the amount of output that the /v option produces, and the resulting program slowdown, it probably shouldn't be used for normal loop or throughput testing.

### T_USB

T_USB is the device-side part of the reflector test program suite; its source code can be found in /e32test/device/(t_usb.[cpp|h], t_usbco2.cpp).

After starting the program, it prompts the user to choose a maximum size for a single transfer. Note that the maximum transfer size is not the same as the maximum packet size.

++++ Choose max. Transfer Size ++++

'0' - Set up USB device for USBCV

'1' -   32 bytes

'2' - 1024 bytes

'3' -   64 kbytes

'4' -    1 Mbyte

The size '0' option is used to circumvent the dynamic version number checking that the program will otherwise perform (because in cases '1'-'4' it assumes it is running against USBRFLCT). Option '0' sets up a static USB device that can be used for other test purposes, including compliance testing with USBCV. Most items on the program's main menu have no meaning after this option.

Following that, the program prompts the user to choose a bandwidth priority for the interface that is about to be created and over which the tests will be conducted.

++++ Choose Bandwidth Priority ++++

'1' - Economical buffering - default

'2' - More memory than default buffering - Plus1

'3' - More memory than Plus1 buffering - Plus2

'4' - Maximum memory for buffering

The next two configuration menus are displayed, asking for endpoint resource option selection.

++++ Choose Endpoint I/O Transfer Mode ++++

'1' - Interrupt Mode

'2' - DMA Mode (recommended)

++++ Choose Endpoint FIFO Mode ++++

'1' - Normal Buffering Mode

'2' - Double Buffering Mode (recommended)

Then a message prompts the user to start the host side test program USBRFLCT,

>>> START THE USBRFLCT PROGRAM ON THE HOST SIDE NOW <<<

and eventually, after successful enumeration by the host and configuration through USBRFLCT, the program's main menu is presented.

++++ Select Program Option ++++

'L'oop test

Loop test with data 'C'ompare

'R'eceive-only test (we receive, host transmits)

'T'ransmit-only test

Receive and 'P'ut (write) to File

Transmit and 'G'et (read) from File

Signal Remote-'W'akeup to the host

'S'top current transfer

Re'E'numerate device

'Q'uit this app

Here one of six test modes can be selected and started. The mode selected should, of course, correspond with the mode the host-side program is running in: device-side Receive matches host side Transmit, etc.

As the main test modes have already been described in the USBRFLCT section, here only some remarks specific to the device side follow.

The ‘Loop with Compare’ test (‘C’) is just like the normal Loop one (‘L’), only that here the data received is compared against what was previously sent, and so any transmission errors can easily be detected and flagged. The reason for having a non-comparing mode is simply that it is quicker, as the byte-by-byte checking of the compare operation can slow down the program operation considerably (especially towards the larger transfer sizes). The non-comparing mode does however also some checking: it compares the received transfer number (4 bytes at the beginning of each buffer) against the expected value.

The standard ‘Receive-only’ test mode (‘R’) immediately discards the data received, and the standard ‘Transmit-only’ mode (‘T’) creates the data to be sent on the spot and ‘out of thin air.’ This not very realistic use case is to be preferred when trying to assess the maximum throughput rate of the link. However, when trying to test a scenario that is more likely to resemble a real-world application, the two file I/O test modes can be used. Here, in the receive mode (‘P’) the data is written to a file, and in the transmit mode (‘G’) the data to be sent is read from a file (which is then created on the fly). In both cases the user is first prompted to choose a drive letter out of a list of those that are available.

With the ‘S’ key an ongoing transfer can be stopped at any time, ‘W’ issues a Remote Wake-up signal to the host (which may be needed when running USBCV, for instance), and ‘E’ will re-enumerate the USB device by first disconnecting it from and then reconnecting it to the bus.

Pressing <ESC> or ‘Q’ will quit the program at any time, after tearing down the USB device configuration and freeing all used resources.

## T_USB_WIN and T_USB_DEVICE

These are the host- and device-side parts, respectively, of the new comprehensive USB Client Test Framework which is now available in Symbian OS versions 9.4 and higher. For a detailed description of that test framework see the separate document [5].

## USBIOAPP

The USBIOAPP program is a Win32 GUI application provided by Thesycon and works, like USBRFLCT, on top of the USBRFLCT.SYS driver (aka USBIO.SYS). Its source code is part of the USBIO development kit installation, and a ready-to-run binary (USBIOAPP.EXE) can be found in the same directory as USBRFLCT and the host-side drivers, /e32test/win32/usbrflct_distribution/.

USBIOAPP can be used to exercise and test the low-level functionality of an attached USB device, in this case the Symbian OS device running the USB device stack including the PSL port.

Here is an example use:

- Connect the Symbian OS USB device to a PC on which the USBRFLCT driver has been installed.

- Start T_USB on the device to create a USB device (use option '0' for the transfer size).

- Launch USBIOAPP on the PC.

- After pressing the ‘Scan for USBIO devices’ button, the device should be detected by USBIOAPP.

- The USB device can now be exercised and tested using the numerous options offered by USBIOAPP.

No further description of the program is given at this point, however the USBIO manual has a short chapter on it. It can be found at http://www.thesycon.de/usbio/usbioman.pdf.

## USBCV (USB Command Verifier)

This test program was written and is maintained by USB Implementers Forum, Inc, (also referred to as the “USB Forum” or “USB-IF”). It can be used to check compatibility of the USB device with the USB specification (currently version 2.0). It contains a collection of different test suites, but the only tests that are of interest regarding the PSL port are the “Chapter 9 Tests”. Even though most of the Chapter 9 request processing is done by the PIL, the tests can help to uncover potential problems with Ep0 controlling PSL code. The tests will also show whether or not the PSL and the PIL work together without problems.

This program currently (R1.3.2) runs on Windows XP SP2 (English Version) only. The current version of the package can always be found here:

http://www.usb.org/developers/tools/.

## USBMSAPP (USB Mass Storage Application)

This is not really a dedicated test program, but can be used to easily verify the proper operation of the USB driver stack. The text shell application usbmsapp.exe is maintained by the Base team, and the source code for it can be found under /e32utils/usbmsapp. Since it is not a test program, USBMSAPP is not normally included in ROMs with any of the e32tests/f32tests/alltests rom.bat options. Instead, in order to include it, the command line macro ROMOPT_USBMSAPP needs to be defined as shown below (to be issued as usual in /e32/rombuild):

rom --variant=??hrp --inst=armv5 --build=urel --type=alltests --symbol --define=ROMOPT_USBMSAPP

Here is another example command line for building a Mass Storage ROM, this time adding also the defines for HSUSB support, FAT32 support in the fileserver, and SD card support in the media driver:

rom --variant=??hrp --inst=armv5 --build=udeb --type=alltests --symbol --define=USE_FAT32 --define=USE_SDIO_SD_MMC --define=USE_HSUSB --define=ROMOPT_USBMSAPP

The Mass Storage application is started in Eshell with a drive letter as a parameter:

C:\> usbmsapp d:

The drive associated with that letter is then mounted with the Mass Storage file system ('MSFS') and so should become visible as a connected R/W disk medium on the USB host computer.

Note: The C: drive and other internal drives may not be mounted for USB Mass Storage. The system only operates with media marked as 'removable'. Internal drives made available for USB Mass Storage must be treated as removable for the purposes of Platform Security.

# Further Information

## People

| Role | Person / People |
| --- | --- |
| Contributor(s) | Robert Jackman, Steve Elliot |
| Reviewer(s) | Robert Jackman, Dale Self, Peter Scobie |

## References

| No. | Document Reference | Version | Description |
| --- | --- | --- | --- |
| [1] | http://www.usb.org/developers/docs/usb_20.zip | 2.0 | Universal Serial Bus Specification |
| [2] | SGL.GT0145.110 | 3.2 | USB Base Driver API on Symbian OS (base/documentation/Base_USB_Driver_API.doc) |
| [3] | http://www.thesycon.de/usbio/eng/usbio.htm | - | USBIO Software Development Kit for Windows |
| [5] | - | 0.3 | Base How To Use Automated USB Tests (/base/documentation/Base_How_To_Use_Automated_USB_Test.doc) |

## Open Issues

None.

## Glossary

The following technical terms and abbreviations are used within this document.

| Term | Definition |
| --- | --- |
| ASSP | Application-Specific Standard Part. An integrated part (integrated circuit) consisting (here) of CPU, MMU, cache and peripherals. Designed and marketed by a silicon vendor and intended to be used in a class of devices. |
| BR | Break Request (in Symbian OS) |
| “Chapter 9” | Chapter 9, “USB Device Framework”, of the USB Specification. Describes Ep0 behaviour, device states, device requests and much else. Chapter 9 and Chapter 7 should be thoroughly understood before attempting to port the Client Controller. |
| DFC | Delayed function call. A call-back function, normally added as part of an ISR to a queue of DFC objects where the function is not called until the operating system is in a known stable state. |
| DMA | Direct Memory Access. A mechanism allowing data to be transferred between a peripheral and a memory buffer or between two memory buffers without the direct involvement of the main system processor - freeing the latter for other tasks. |
| DUT | Device Under Test |
| E32 | The software component containing implementations of the kernel for a number of hardware platforms, associated device drivers and the Symbian OS user library. |
| EKA | EPOC Kernel Architecture; there are EKA1 and EKA2 |
| Ep0 | Endpoint Zero. The USB device’s special control endpoint. |
| HRP | Hardware Reference Platform |
| HSUSB | High-speed USB |
| IP | Intellectual Property |
| ISR | Interrupt Service Routine. A short software function called in response to an interrupt and which services that interrupt. |
| Kernel Extension | A specifically nominated DLL which is executed during kernel boot. Used for kernel side code that isn’t part of the kernel itself and which can be replaced or omitted on a per-ROM basis as hardware or functionality requires. |
| LDD | Logical Device Driver. The higher layer of abstraction within the Symbian OS device driver framework which implements common functionality among differing pieces of hardware of one type. |
| OTG | USB On-the-go; a supplement to the USB 2.0 specification |
| Packet | The basic USB transfer unit, consisting of zero or more bytes. |
| PDD | Physical Device Driver. The lower layer of abstraction within the Symbian OS device driver framework which implements functionality that is specific to a particular piece of hardware. |
| PIL | Platform Independent Layer of the Controller |
| PSL | Platform Specific Layer of the Controller |
| UDC | USB Device Controller |
| USB | Universal Serial Bus. An external peripheral bus standard that supports data transfers rates of up to 12 Mbps (USB version 1.1), or 480 Mbps (USB version 2.0, ‘high-speed’). |
| USB Host | The USB system consists of single USB Host controller connected to a number of USB peripheral devices. The Host is the bus master - initiating all transactions over the bus. |
| USB Device | A USB peripheral device. It has to be connected to a USB host and passively responds to requests addressed to it from the host. |
| USB (Driver) Stack | here: The entirety of the USB device driver software in Symbian OS, comprising USB PDD (PSL + PIL) and USB LDD. |
| ZLP | Zero Length Packet |

## Document History

The 'Re-enumerate' command might behave strangely, and it is actually not recommended that it is used at all – it's not testing any functionality that's not tested otherwise anyway. The command will probably be removed in future versions of T_USB.