Contents

1	Introduction	3

1.1	Purpose and Scope	3

2	Brief review of eMMC memory card systems	3

3	Porting the eMMC Controller	4

3.1	DMMCStack derived class	4

3.1.1	DMMCStack::MachineInfo()	4

3.1.2	DMMCStack::IssueMMCCommandSM()	4

3.2	TMMCardControllerInterface Derived Class	4

3.2.1	TMMCardControllerInterface::NewMediaChange()	4

3.3	Customising the Embedded MMC Partition Scheme	5

3.3.1	Introduction	5

3.3.2	Reference eMMC partition scheme	5

3.3.3	Implement custom partition scheme	5

3.4	Sleep Mode	10

3.4.1	Introduction	10

3.4.2	Implement separate VccCore and Vcc I/O PSU’s	11

3.4.3	Implement separate Asynchronous power up for VccCore supply	13

3.4.4	Set Idle and Sleep Timeouts	14

4	Testing the port	15

4.1	Unit tests	15

5	Further Information	16

5.1	People	16

5.2	References	16

5.3	Glossary	17

5.4	Document History	18

# Introduction

## Purpose and Scope

The software MultiMediaCard (MMC) card controller provides support for MultiMediaCards within the E32 kernel. This supplement document together with the MultiMediaCard controller section of the base porting guide [R1] describes how to port this controller for a new eMMC hardware interface.

The reader is assumed to be familiar with the following documentation:

- Base Porting Guide – Multimedia Card (MMC) Controller [R1]

- MMCA Specification v4.3 [R2]

# Brief review of eMMC memory card systems

eMMC is a mass data storage device that utilizes the MultiMediaCard (MMC) protocol interface. It features low cost, small size, Flash-technology independence, and high data throughput. These features make eMMC ideal for smart phones and other mobile devices.

An eMMC device includes a Flash memory component and a controller on an advanced 10-signal bus.

# Porting the eMMC Controller

In addition to those functions described in the base porting guide [R1] which need to be implemented for the PSL, the following sections details additional activities required for the porting of embedded MMC devices.

## DMMCStack derived class

### DMMCStack::MachineInfo()

The following flag must be added to indicate that an Embedded or internal memory device is in use:

aMachineInfo.iFlags |= TMMCMachineInfo::ESlot1Internal;

### DMMCStack::IssueMMCCommandSM()

As part of the MMCA v4.3 Specification [R2] the CMD5 and CMD23 protocol commands have been defined or modified; the PSL implementation of IssueMMCCommandSM() should not modify the of arguments or reject these protocol commands.

#### CMD5

CMD5 (SLEEP_AWAKE) has been defined within the MMCA v4.3 specification, it is used to toggle devices between SLEEP and STANDBY states, setting bit 15 indicates that a device should transition from STANDBY to SLEEP state; clearing bit15 indicates that the device should transition from SLEEP to STANDBY state.

Note. Once in SLEEP state devices will only respond to either CMD5 or CMD0 (GO_IDLE_STATE), all other commands are ignored.

#### CMD23

CMD23 (SET_BLOCK_COUNT) has been modified as part of the reliable write functionality, CMD23 will now be sent with bit31 set to indicate that the following write request is to be treated as a reliable write operation.

## TMMCardControllerInterface Derived Class

### TMMCardControllerInterface::NewMediaChange()

Embedded/Internal media cannot be removed; there is no longer a requirement for media change functionality. TMMCardControllerInterface::NewMediaChange() maybe modified to return the class DMMCEmbeddedMediaChange, which implements basic media change functionality such that media is always reported as present.

DMMCMediaChange* TMMCardControllerInterface::NewMediaChange(TInt aMcId)

/**

* Create a new platform-specific media change

*

* @param aMcId identifier for new media change object

* @return The new media change object (or failure)

*/

{

return new DMMCEmbeddedMediaChange(aMcId);

}

## Customising the Embedded MMC Partition Scheme

### Introduction

For Embedded MMC devices the MMC media driver has been modified to make use of a separate static library that defines the partitioning scheme used with Embedded MMC Devices, legacy support for removable media (DOS MBR partitioning) is still retained within the MMC media driver.

MEDMMC.PDD

EMMCPTN.LIB

### Reference eMMC partition scheme

The default reference eMMC partition scheme is based upon the DOS MBR partition scheme; however this scheme only supports 4 partitions which can be inadequate for some platform configurations. The scheme has therefore been modified to remove the Code Area and Optional Disk Signature fields and the table of partitions has been extended downwards. The diagram below reflects this layout:

| Address | Description | Size in Bytes |
| --- | --- | --- |
| 0 | Nulls; 0x0000 | 14 |
| 14 | Table of primary partitions (Up to 31 16-byte entries, IBM Partition Table scheme) | 496 |
| 510 | MBR signature; 0xAA55 | 2 |
| MBR, total size: 14 + 496 + 2 = |  | 512 |

Figure 1 – Non-Compliant MBR layout

The emmcptn library may be redefined and replaced on a per platform basis, the follow sections discuss the interfaces that must be satisfied when implementing the partition library.

### Implement custom partition scheme

An Abstract class has been defined within emmcptn.h as the interface for eMMC partitioning schemes:

class DEMMCPartitionInfo : public DBase

/**

* Base Abstract class for classes that define a partitioning scheme for Embedded MMC devices

*

* @publishedPartner

* @released

*/

{

public:

/**

* Initialise the Partition Object.

* Called as part of the creation process for the MMC Media driver.

* Initialisation can include activities such as Memory structure allocation and

* creation of any required driver session/handles.

* @param aDriver Owning DMediaDriver class

* @return KErrNone if successful, standard error code otherwise.

*/

virtual TInt Initialise(DMediaDriver* aDriver) =0;

/**

* Read the partition information for the media.

* Called as a child function of the PartitionInfo() method.

* @see DMmcMediaDriverFlash::PartitionInfo()

* @param anInfo An object to store the partition information.

* @param aCallBack Function to be called upon completion.

* @return KErrNone if successful, standard error code otherwise.

*/

virtual TInt PartitionInfo(TPartitionInfo& anInfo, const TMMCCallBack& aCallBack) =0;

/**

* Returns partition specific drive capability attributes.

* Called as a child function of the Caps() method.

* @see DMmcMediaDriverFlash::Caps()

* @param aDrive Local drive to be queried.

* @param aInfo Media drive capability object to be populated.

* @return KErrNone if successful, standard error code otherwise.

*/

virtual TInt PartitionCaps(TLocDrv& aDrive, TDes8& aInfo) =0;

/**

* Factory Method that returns an instance of a DEMMCPartitionInfo class.

* @return Newly created DEMMCPartitionInfo object.

*/

IMPORT_C DEMMCPartitionInfo* CreateEmmcPartitionInfo();

All custom partition scheme implementations must be derived from this base class.

The following sections describe the partition scheme methods that need to be defined:

#### CreateEmmcPartitionInfo()

The CreateEmmcPartitionInfo() method is called by the MMC media driver at initialisation to instantiate an eMMC partition scheme object for use with partition queries.

EXPORT_C DEMMCPartitionInfo* CreateEmmcPartitionInfo()

{

return new DLegacyEMMCPartitionInfo;

}

This method should return a pointer to a newly created object of the DEMMCPartitionInfo derived class.

#### Initialise()

The Initialise() method is called by the MMC media driver at initialisation to allow the eMMC partition scheme object to allocate/create/acquire any resources required for the implementation.

TInt DLegacyEMMCPartitionInfo::Initialise(DMediaDriver* aDriver)

{

iDriver = aDriver;

DPBusPrimaryMedia* primaryMedia = (DPBusPrimaryMedia*)(iDriver->iPrimaryMedia);

DMMCSocket* socket = ((DMMCSocket*)(primaryMedia->iSocket);

if(socket == NULL)

return(KErrNoMemory);

DMMCStack* stack = socket->Stack(KStackNumber);

iCard = stack->CardP(primaryMedia->iSlotNumber);

iSession = stack->AllocSession(iSessionEndCallBack);

if (iSession == NULL)

return(KErrNoMemory);

iSession->SetStack(stack);

iSession->SetCard(iCard);

TInt bufLen, minorBufLen;

stack->BufferInfo(iIntBuf, bufLen, minorBufLen);

return(KErrNone);

}

The example code initialisation sequence above includes the creation of a MMC Stack session such that ‘read’ commands can be issued to the media.

#### PartitionInfo()

The PartitionInfo() method will be called by the MMC media driver for those sockets that are marked as internal (meaning embedded MMC).

A reference to a TPartitionInfo object is passed as a parameter; it is this object that is to be populated with partition information.

Partition information includes:

- Partition type - Types reported must use the constants defined within partitions.h.

- Address – Partition start location on media in Bytes

- Length – Size of Partition on media in Bytes

A TMMCCallBack reference is also passed as a parameter; once the TPartitionInfo object is populated the TMMCCallBack object is called to inform the MMC media driver that the partition information is available. For Synchronous implementations (i.e. hardcoded tables) this may be called back from within PartitionInfo(), Otherwise for asynchronous implementations this will need be called in a completion method.

The example partitionInfo() below is an asynchronous implementation that reads the first 512 byte of the media and then processes the data retrieved.

TInt DLegacyEMMCPartitionInfo::PartitionInfo(TPartitionInfo& anInfo, const TMMCCallBack& aCallBack)

{

iPartitionInfo = &anInfo;

iCallBack = aCallBack;

// If media driver is persistent (see EMediaDriverPersistent),

// the card may have changed since last power down, so reset CID

iSession->SetCard(iCard);

iSession->SetupCIMReadBlock(0, iIntBuf);

TInt r = iDriver->InCritical();

if (r == KErrNone)

r = iSession->Engage();

if(r != KErrNone)

iDriver->EndInCritical();

return(r);

}

void DLegacyEMMCPartitionInfo::DoSessionEndCallBack()

//

// Block Read Complete, if no errors call DecodePartitionInfo()

//

{

iDriver->EndInCritical();

TInt r = iSession->EpocErrorCode();

if (r == KErrNone)

r = DecodePartitionInfo();

iDriver->PartitionInfoComplete(r == KErrNone ? r : KErrNotReady);

}

TInt DLegacyEMMCPartitionInfo::DecodePartitionInfo()

//

// Decode partition info that was read into internal buffer

//

{

TUint partitionCount=iPartitionInfo->iPartitionCount=0;

TMBRPartitionEntry* pe;

const TUint KMBRFirstPartitionOffsetAligned = KMBRFirstPartitionOffset & ~3;

TInt i;

// Read of the first sector successful so check for a Master Boot Record

if (*(TUint16*)(&iIntBuf[KMBRSignatureOffset])!=0xAA55)

return KErrCorrupt;

memmove(&iIntBuf[0], &iIntBuf[2],	KMBRFirstPartitionOffsetAligned +

KMBRMaxPrimaryPartitions * sizeof(TMBRPartitionEntry));

...

for (i=0, pe = (TMBRPartitionEntry*)(&iIntBuf[KMBRFirstPartitionOffsetAligned]);

pe->iPartitionType != 0 && i < KMaxPartitionEntries; i++,pe--)

{

...

// FAT partition?

if (pe->IsValidDosPartition() || pe->IsValidFAT32Partition())

{

SetPartitionEntry(&iPartitionInfo->iEntry[partitionCount],

pe->iFirstSector,

pe->iNumSectors );

partitionCount++;

}

// ROM partition?

else if (pe->iPartitionType == KPartitionTypeROM)

{

SetPartitionEntry(&iPartitionInfo->iEntry[partitionCount],

pe->iFirstSector,

pe->iNumSectors);

&iPartitionInfo->iEntry[partitionCount].iPartitionType =

pe->iPartitionType;

partitionCount++;

}

}

...

iPartitionInfo->iPartitionCount=partitionCount;

iPartitionInfo->iMediaSizeInBytes=iCard->DeviceSize64();

//Notify medmmc that partitioninfo is complete.

iCallBack.CallBack();

return(KErrNone);

}

#### PartitionCaps

The PartitionCaps() method is called by the MMC media driver as part of the ::Caps() method, this method allows any partition scheme specific capabilities to be added/removed/altered to the Local Drive Capabilities before they are reported.

TInt DLegacyEMMCPartitionInfo::PartitionCaps(TLocDrv& aDrive, TDes8& aInfo)

{

TLocalDriveCapsV6Buf& Info = static_cast< TLocalDriveCapsV6Buf&> (aInfo);

// is this query for the ROFS partition?

if (aDrive.iPartitionType == KPartitionTypeRofs)

{

aInfo.iFileSystemId = KDriveFileSysROFS;

aInfo.iMediaAtt&= ~KMediaAttFormattable;

aInfo.iMediaAtt|= KMediaAttWriteProtected;

}

// is this query for the ROM partition?

if (aDrive.iPartitionType == KPartitionTypeROM)

{

aInfo.iFileSystemId = KDriveFileNone;

aInfo.iMediaAtt&= ~KMediaAttFormattable;

aInfo.iMediaAtt|= KMediaAttWriteProtected;

}

return KErrNone;

}

The example code demonstrates that for a local drive capability query of a either a ROFS or ROM partition, this scheme will report these partitions as Read only.

This can be verified by issuing a DRVINFO command within tshell and noting the returned attribute information about each drive. Using the example code above and querying the ROM drive would return local drive attributes containing KMediaAttWriteProtected.

The reference eMMC partition scheme/library is included by default into all ROM’s; to remove the default implementation the macro USE_CUSTOM_MMC_PARTITION must be defined within the platform’s header.iby. A custom eMMC partition scheme may then added to the platform kernel.iby.

extension[VARID]=KERNEL_DIR\DEBUG_DIR\DPLAT_EMMCPTN.DLL \sys\bin\EMMCPTN.DLL

## Sleep Mode

### Introduction

A new “Sleep” state has been introduced as part of the MMC v4.3 specifications to aid in the reduction of power consumption. See specification [R2] section 7.5.12.

eMMC devices have been designed to have separate supplies for control logic (VccQ) and memory field (VccCore) unlike legacy removable media which only have a single Vcc for both.

Driver request

Driver request

Sleep Timer Expired

Issue CMD5

Power Off VccCore

SLEEP

STANDBY

Power Up VccCore

Issue CMD5

Power OFF

Power Up VccQ

Power Up VccCore

Issue Driver Request

Idle Timer Expired

Power Down Stack

Power Off VccQ

Full Initialisation

CMD0->CMD1->….

Media may be switched between SLEEP and STANDBY modes by use of CMD5 (SLEEP/AWAKE).

The MMCA specification also recommends that media power consumption may be further reduced by removing the VccCore supply (Memory field voltage) once in SLEEP mode; Note VccQ is maintained (Control Logic Voltage).

The Legacy MMC implementation provided a single ‘idle’ timer, which upon timeout the MMC media would be entirely powered down. Support for a secondary ‘sleep’ timer has been introduced; A 2 stage power-down scheme may now be implemented; for example after 30 seconds of inactivity media is placed into SLEEP mode then after 5 minutes the media is powered off.

To enable the use of Sleep mode with eMMC devices, the PSL must be modified in the following manner:

- Implement separate VccCore & Vcc I/O PSU’s

- Implement separate Asynchronous power up for VccCore supply

- Set Idle and Sleep Timeouts

### Implement separate VccCore and Vcc I/O PSU’s

The task will detail the creation/reporting and basic operation of the VccCore PSU object.

#### VccCore PSU

As part of the tasks detailed with the MultiMediaCard controller section of the base porting guide a PSU object must be created, for eMMC devices this existing PSU will represent the VccQ (or I/O Vcc) supply. To support sleep mode an additional PSU object must be created to represent the VccCore (or Memory) supply.

DMMCSocket

DMMCPsu

(Vcc Core)

DMMCPsu

(VccQ I/O)

The Vcc Core PSU class can be implemented following the same guidelines as detailed for the DMMCPsu derived class in the MultiMediaCard controller section of the base porting guide [R1].

void DPlatMMCPsuVccCore::PsuInfo(TPBusPsuInfo& anInfo)

{

…

anInfo.iNotLockedTimeOut = 0;

anInfo.iInactivityTimeOut = 0;

return;

}

At this stage of porting activity it is recommended that the attributes iNotLockedTimeOut & iInactivityTimeOut for the VccCore PSU object be set to 0 to disable idle/Sleep timeouts. This will allow operation of the eMMC device to be tested without the sleep mode functionality activated.

#### Registering the VccCore PSU

The TMMCardControllerInterface class has been extended to provide an API for creation and registering of the VccCore PSU object:

/**

Creates a platform specific VccCore power supply unit object.

The function is called from TMMCardControllerInterface::Create(), and

an implementation can be provided by the platform specific layer in

the Variant DLL.

@param aVccCoreNo Power supply unit number (must match paired Vcc unit number).

@param aMcId      Associated media change number.

@return A pointer to the new power supply unit object.

@see Create()

@see MediaChangeID()

@see VccID()

*/

virtual DMMCPsu* NewVccCore(TInt aVccCoreNum, TInt aMcId);

The function should create, and return a pointer to, an instance of a DMMCPsu derived class for VccCore. The function is called from TMMCardControllerInterface::Create().

The Power Supply Unit (PSU) number and the media change number should be forwarded to the DMMCPsu constructor in your class constructor's ctor list.

Note:

- Vcc and VccCore PSU objects are paired within the MMC stack they are both referenced by the same socket.

- The PSU number can fall into the range 0 to KMaxPBusVccs, and is the value returned by VccID(); it must match the value used with NewVcc().

- The media change number can fall into the range 0 to KMaxMediaChanges, and is the value returned by MediaChangeID().; it must match the value used with NewVcc().

- This function is only called for sockets that are associated with MultiMediaCard devices as reported by the function IsMMCSocket().

#### VccCore Operation

The DoPowerUpSM() state machine is responsible for performing the necessary PSL level actions associated with powering up the bus. For eMMC devices this requires both the Vcc and the VccCore supplies to be fully powered before stack operation may begin.

This state machine must be modified to power up the VccCore PSU in addition to the Vcc PSU.

TMMCErr DPlatMMCStack::DoPowerUpSM()

{

…

if (MMCSocket()->iVccCore)

{

if ( MMCSocket()->iVccCore->SetState( EpsuOnFull ) != KerrNone )

{

return KMMCErrHardware;

}

}

…

#### Test

Once this porting task has been completed it is recommended to run regression tests (see section 4) to verify PSL and device operation.

### Implement separate Asynchronous power up for VccCore supply

An additional MMC Stack state machine is required to permit an independent Power up the VccCore PSU, this is required as part of the process to “Wake” an eMMC device from SLEEP state (i.e. transition from SLEEP to STANDBY state).

#### Implement DoWakeUpSM Interface method

To aid in future proofing the DoWakeUpSM interface has been implemented utilising the interface mixin class design pattern. The PSL DMMCStack derived Class must be modified to implement the (new) DMMCStack::MDoWakeUp interface method.

/**

* An optional interface State machine implemented by the

* PSL for handling asynchronous VccQ powerup

* @see KinterfaceDoWakeUpSM

*/

class MDoWakeUp

{

public:

virtual TMMCErr DoWakeUpSM()=0;

};

The PSL DMMCStack derived class must be modified to inherit from the MdoWakeUp interface class in addition to the DMMCStack class.

class DPlatMMCStack : public DMMCStack::MDoWakeUp, public DMMCStack

This inheritance is necessary to allow the use of MMC State machine and its associated macros (i.e. SMF_BEGIN, SMD_STATE, SMF_END etc) when implementing the DoWakeUpSM() method.

The GetInterface() method must be defined for the PSL DMMCStack derived class, the method takes a TinterfaceId parameter to identify the interface required by the PIL DMMCStack, in this case it is required for the MdoWakeUp interface.

Void DPlatMMCStack::GetInterface(TinterfaceId aInterfaceId, Minterface*& aInterfacePtr)

{

if (aInterfaceId == KinterfaceDoWakeUpSM)

{

aInterfacePtr = (DMMCStack::Minterface*) this;

}

}

Note in the example above that a pointer to the DPlatMMCStack is returned, within the DMMCStack this pointer is cast to the MdoWakeUp interface before making a call to the DOWakeUpSM() method.

#### Implement DOWakeUpSM() method

This method should be implemented to power up the VccCore PSU. This maybe performed in either an asynchronous or synchronous manner; however the state machine may not exit until the VccCore supply is stable.

This example implementation demonstrates an asynchronous power up of the VccCore PSU.

TMMCErr DPlatMMCStack::DoWakeUpSM()

{

enum states

{

EstBegin = 0,

EstPowerOn,

EstEnd

};

// Set up the state machine switch statement inside an infinite loop.

SMF_BEGIN

__KTRACE_OPT(KPBUS1, Kern::Printf(“Dplat:DoWakeUpSM: BEGIN”));

// block before calling PSU layer in case it calls

// back immediately with an unblock

BlockCurrentSession(KMMCBlockOnASSPFunction);

if (MMCSocket()->iVccQ)

{

if (MMCSocket()->iVccQ->SetState( EpsuOnFull ) != KerrNone )

{

Kern::Printf(“VccQ error”);

return KMMCErrHardware;

}

}

// Wait for the PSU to report that the

// VccQ supply was powered up successfully.

SMF_WAITS(EstPowerOn);

SMF_STATE(EstPowerOn)

__KTRACE_OPT(KPBUS1, Kern::Printf(“Dplat:DoWakeUpSM: EstPowerOn”));

// End the loop and switch statement.  Do not put any code after this

// as this macro contains the return from the function.

SMF_END

}

#### Test

Although this functionality has not been enabled as part of this task it is recommended to run regression tests (see section 4) to verify PSL and device operation.

### Set Idle and Sleep Timeouts

As stated in a previous section the Vcc PSU timeout is unchanged in functionality and is still associated with the complete power down of the MMC Controller. As such the definitions of iNotLockedTimeOut & iInactivityTimeOut are unchanged with use of the Vcc PSU.

The VccCore PSU is associated with the ‘SLEEP’ mode functionality of eMMC devices, respectively the attribute iInactivityTimeOut for the VccCore PSU represents the time in seconds before an eMMC device is placed into SLEEP state (once in SLEEP State VccCore will be switched off). The attribute iNotLockedTimeOut is not utilised.

The exact value of timeouts for Vcc and VccCore will need to be determined on a platform by platform basis; however note that if a greater timeout is given for VccCore than Vcc, Sleep state will never be utilised as both timers are disabled at complete MMC controller shut down.

#### Test

Once this porting task has been completed it is recommended to run regression tests (see section 4) to verify PSL and device operation.

In addition PSU operation should be validated; this can be accomplished by 2 different methods:

1.) Debug Output – Enabling the kernel logging for MMC protocol stack (kernaltrace 0x80001000) and ensure that calls to power up/down Vcc and VccCore PSU’s occur at the correct time intervals. Note. This method of testing does not guarantee correct operation of hardware.

2.) Monitor Voltage levels – Attach probes to Vcc and VccCore supplies to the eMMC device (where possible); monitor usage to ensure Vcc and VccCore levels are manipulated at the correct time intervals.

# Testing the port

## Unit tests

The process of testing a port of the eMMC card controller is the same as described for the MultiMediaCard controller – see the base porting guide [R1].

# Further Information

## People

| Role | Person / People |
| --- | --- |
| Reviewers | Stefan Williams, Fabian Gortz, Michael Moate |
| Contributors | Neil Clifford |

## References

| No. | Document Reference | Version | Description | Source |
| --- | --- | --- | --- | --- |
| [R1] | - | - | Base Porting Guide – Multimedia Card (MMC) Controller | Symbian OS Library |
| [R2] | JESD84-A43 | 4.3 | Embedded MultiMediaCard (eMMC) eMMC/Card product Standard, High Capacity, including Reliable Write, Boot and Sleep Modes | JEDEC |

## Glossary

| Term | Definition |
| --- | --- |
| E32 | The software component containing implementations of the kernel for a number of hardware platforms, associated device drivers and the User Library. |
| eMMC | Embedded MultiMediaCard |
| F32 | The software component containing the file server, associated file systems and the loader. |
| MMCA | MultiMediaCard Association. An international organization created to promote the adoption of the MultiMediaCard as the storage standard for telecommunications products. |
| MultiMediaCard | A data storage device providing small-size, high capacity storage to handheld communication devices. |
| MultiMediaCard stack | A set of MultiMediaCards connected to the same bus. |
| PIL | Platform Independent Layer. Software layer of the MultiMediaCard / SD card controller which is generic to all MultiMediaCard / SD card platforms and therefore independent of the hardware interface. (See also PSL). |
| PSL | Platform Specific Layer. Software layer of the MultiMediaCard / SD card controller which is specific to the hardware interface. (See also PIL). |
| PSU | Power Supply Unit |

## Document History

| Date | Version | Status | Author | Description |
| --- | --- | --- | --- | --- |
| 29/06/2009 | 1.0 | Issued | Neil Clifford | First Issue |
| 30/03/2009 | 0.1 | Draft | Neil Clifford | First Draft |