.

Design Document for “PREQ1480 - Naiad System Quality- Performance, ROM and RAM targets” ECOM Boot time performance improvement

| Status: | Issued |
| --- | --- |
| Version: | 1.0 |
| Author: | Husien Hong |
| Reviewers: | Simon Conway |
| Approver: | Stephen Mansfield |
| Team/Department : | System Libraries/Core Development |

Contents

1	Introduction	3

1.1	Purpose and Scope	3

2	Design Overview	3

3	Detailed Design	3

3.1	Optimize persisted DAT file structure	3

3.2	Remove the IDX feature and merge multiple drive list handling inside ECOMSERVER	4

3.3	Defer DAT file persistence for Read only Internal drives to Non critical stage	4

3.4	Improving the performance of client side API	4

3.5	General improvement of structure used inside ECOM	5

4	Test specification	5

4.1	Unit Testing	5

4.2	Component Testing	6

4.3	Performance Testing	6

5	Further Information	6

5.1	References	6

5.2	Open Issues	6

5.3	Glossary	7

5.4	People	7

5.5	Document History	7

# Introduction

## Purpose and Scope

This document describes the changes required to the ECOM component in order to satisfy “PREQ1480- Naiad System Quality- Performance, ROM and RAM targets”. This PREQ is required for 9.4 (Naiad).

The target audience that this document is intended includes the TA and engineers within System Libraries.

# Design Overview

One of the purposes of this PREQ is to improve the boot time performance in Symbian OS. As ECOMSERVER is one of the main components that are started during boot time, improving the performance of ECOMSERVER total initialisation time will effectively result in faster boot time. This document will further elaborate the performance actions that have been identified in the IA report [see R1]

Based on the analysis done as part of the IA report, 4 actions have been identified:

- Optimize persisted DAT file structure

- Defer DAT file persistence for Read only Internal drives to Non critical stage

- Improving the performance of client side API

- Remove IDX feature and merge multiple drive list handling inside ECOMSERVER

The actions above do not only focus on the initialisation time but also on other area such as the peak and committed heap usage in the server side and also the disk space used to hold the server private internal files.

# Detailed Design

## Optimize persisted DAT file structure

Target Platform

ROM with SPI disabled and Emulator

Target Improvement area

- Reduce disk space through reduced size of DAT file

- Reduce peak memory usage inside ECOMSERVER during reading of the DAT file

- Reduce boot time through reducing ECOMSERVER initialisation time as a result of reading/writing smaller DAT file

Design Idea

To reduce the size of persisted DAT file, we need to fully understand and consider which persisted information is redundant, duplicate and the structure of the class containing this information. We have identified the following 2 classes where some of the persisted information is not necessary:

CRegistryData::CDllData

Improvements on this class are:

- No need to persist the path of the DLL(“<drive>:\sys\bin\”) and the DLL extension(“.DLL”) in the DLL name variable. Instead, we just store the DLL name and then in its parent class(CDriveData) we add a new TFileName member which contains pregenerated apth in the form (<drive>:\sys\bin\), and any time when we need to reference the full path of the DLL, we can just append the DLL name to this path and use it.

- Replace TUidType with TUid as only the third UID of the DLL is unique, this also means we now only need to persist the single TUid and generate the TUidType whenever this is needed.

CRegistryData::CImplementationInformation

Improvements on this class are:

- Combine the 3 TBool(iDisabled,iRomOnly and iRomBased) into a single 8 bit Integer and hence only persist this 1 byte rather than previous 12 byte.

- In the server side when persisting into DAT file, the drive info of this class is not required as this can be deduced from the DAT file itself. However this is required when streaming this class to the client side during ListImplementations call. To do this we use a variable to indicate the purpose of this streaming to separate out the two different behaviours. This also applies to the VID where it will be retrieved during call to SaveSecurityInfo and hence there is no need to persist this to DAT file.

Current prototype shows that this results in a reduced DAT file in Techview from the original 56K down to 40K. Not only will this improvement save the disk space, it also speeds up by reading smaller file and reduces the size of the committed heap in the server side.

## Remove the IDX feature and merge multiple drive list handling inside ECOMSERVER

Target Platform

All, ROM with both SPI and non-SPI

Target Improvement Area

Reduce the critical stage boot time of ECOM

Design Idea

The idea here is to remove reading and writing of IDX file, currently this is recreated every time a drive is discovered successfully. Originally the purpose of the IDX file is to store all the drive entries that contain a ECOM DAT file but this is not efficient as the IDX file itself might be corrupted plus the reading and writing to the disk makes it not suitable for boot time purpose. To achieve the same functionality, we are going to use the BaflUtils::GetDiskLists to retrieve a list of valid drives in a device and then trying to open the DAT file (In practice the list of drives in a real device should not be more than 10) This list is then cached and then further reused when constructing the CDirChangeNotifier in the discoverer class.

## Defer DAT file persistence for Read only Internal drives to Non critical stage

Target Platform

ROM with SPI Disabled, Emulator .

Target Improvement Area

Reduce the critical stage boot time of Ecom

Design idea

The idea is to simply defer the persisting of ECOM dat file from the critical start-up stage to the Non-critical start-up stage. This means reducing the critical start up stage time at the expense of the later non-critical initialisation time.

## Improving the performance of client side API

Target Platform

All, ROM with both SPI and non-SPI

Target Improvement Area

- Reduce boot time through removing unnecessary/redundant information passed through IPC

- Reduce boot time peak memory usage through more efficient use of structure to store ECOM client side information

Design Idea

In the ECOM client side, the structure used to hold the ECOM internal information can be improved to reduce the heap usage and information passed through the IPC boundary. This improvement can be done in three areas:

- Reducing the heap required in the client side by removing some redundant information. The CEComEntry used in the client side does not need to contain the TUidType but we can simply store the 3rd unique TUid. Furthermore the member modified time in this class is never needed in the client side so we can actually configure this member out in the client side using a macro. Finally the member iVersion in the class TUnloadPolicyIndex is never used/ referenced at all( this is likely to be used before 9.1 where the loading of the DLL is done in the server side).

- CLoadManager maintains a list of policy index mapping but this is not ordered and hence a linear search is performed to locate a match. By ordering the policy based on the implementation uid we can use binary search to speed up the search.

- Existing ListImplementations call results in two IPC request to the ECOM server the first one to determine the size of the buffer that the client needs to create to hold the implementation information and the second one is to actually copy the implementation to the client’s pre-created buffer. The improvement here is to try to always set a default size for the implementation buffer so if this is big enough the server can immediately write to this buffer resulting in only 1 IPC call. In the case when the buffer is not big enough, it will indicate to the client side the “required” size and the client side will then create the required buffer before sending another IPC call to collect the implementations. This idea can be further improved by storing the default buffer size as a patchable constant which allows easy rom build time modification.

## General improvement of structure used inside ECOM

Target Platform

All, ROM with both SPI and non-SPI

Target Improvement Area

Reduce boot time peak memory usage through more efficient use of structure to store information in the server side

Design Idea

This section is used to contain minor improvements that can be done in the server side and discovered when analyzing the server code.

- Caching the call to RFs::GetSystemDrive, currently there is at least 5 instances of this call and it is never cached, this should only be called once.

- Removing the use of TFileName in any structure and replace it with a dynamic buffer HBufC. TFileName is TBuf<255> and hence is not efficient.

- Currently the CDllData stores the full path of the DLL(<drive>:\sys\bin\*.DLL), by having a pre-constructed buffer containing the drive and path in the CDriveData, we can remove this drive and path and only construct this information when needed( this is hardly used and only required during getting security info) by appending to the buffer stored in CDriveData.

# Test specification

This section outlines any testing that will be delivered as part of this PREQ. As this is a performance improvement PREQ, we need to ensure there is no regression plus we want improvement otherwise the code churn introduced is not even worth it.

## Unit Testing

Existing unit test will be updated to reflect any changes made in the structure of a class. As all the code changes only involve internal classes and majority of the changes are improvement of the implementation logic there will be minor update of the unit test. Ensuring no regression on existing unit test should be sufficient.

## Component Testing

Similar to unit testing where majority are implementation improvement and involve internal classes only, there will be no new component test. Ensuring no regression on existing component test should be sufficient.

## Performance Testing

Performance testing will be the key component of this test specification.

TARGET AREA

- Boot time(EXISTING TEST)

This will focus on the initialisation time at various stages during ECOM start-up (i.e. critical static, dynamic and non-critical static etc). In particular we will pay most attention to the critical static and our target set will be to achieve at least 10% improvement in this area.

As there is already an existing test (TE_ECOMPERFTEST) that can be used to measure this, the task involve will be to measure the improvement and towards the end to adjust the performance benchmark in the test itself.

- Server peak heap usage(NEW TEST)

This will be a new test case that will be added as part of the existing TE_ECOMPERFTEST to measure the heap usage in the server i.e. the total committed heap usage and how the heap grows at various stage of the ECOM start up. This test will follow existing performance test by having the server storing the various performance figures during the boot-up and the test script will then retrieve these values using a test client side API.

As this will be a new test case, we will first measure and compare the performance figure when our changes are not applied and then set a benchmark for future testing using the modified code.

- Client side request time(EXISTING TEST)

This will focus on the average time taken during the client side API service request during call to CreateImplementation and ListImplementation. Measurement will be performed both in the client and server side using existing TE_ECOMPERFTEST.

TARGET PLATFORM

The performance figure will be obtained for H2 hardware (both SPI and non-SPI) (UREL only) and will be measured across different ROM configurations (RAM, NOR, NAND) and also taking account into the cold boot (1st boot) and warm boot (2nd boot) measurement.

# Further Information

## References

| No. | Document Reference | Description |
| --- | --- | --- |
| [R1] | V:\Dev\TS\TS0017 Base Services\RequirementsAnalysis\ProductRequirements\PREQ1480 - Naiad System Quality- Performance, ROM and RAM targets\ SGL.TS0017.xxx IA Report PREQ1480 v0-2.doc | IA Report |
| [R2] | V:\Dev\TS\TS0017 Base Services\RequirementsAnalysis\ProductRequirements\PREQ1480 - Naiad System Quality- Performance, ROM and RAM targets\ SGL.TS0017.645 PREQ1480 ECOM Boot-up Investigation Report v0-42.doc | ECOM performance investigation report |

## Open Issues

None

## Glossary

| Term | Definition |
| --- | --- |
| SPI | Static Plug-in Information – a concatenation of plug-in resource files present on each internal read only drive |

## People

| Role | Person / People |
| --- | --- |
| Reviewers | Simon Conway, Stephen Mansfield |
| Location | V:\Dev\GT\GT 0304 System Libraries\Technical |

## Document History

| Date | Version | Status | Author | Description |
| --- | --- | --- | --- | --- |
| 2006-12-05 | 1.0 | Issued | Stephen Mansfield | Document issued. |
| 2006-11-20 | 0.4 | For Review | Husien Hong | Adding the new ListImplementation optimisation idea in section 3.4 |
| 2006-11-9 | 0.3 | Draft | Husien Hong | Applying Simon Conway’s comment |
| 2006-10-31 | 0.2 | Draft | Husien Hong | Design ideas plus test specifcation |
| 2006-10-30 | 0.1 | Draft | Husien Hong | First draft |