Alarm Server Configuration How-To Document

| Status: | Issued |
| --- | --- |
| Version: | 1.1 |
| Team/Department : | PIM Application Services |

Contents

1	Introduction	2

1.1	Purpose and Scope	2

2	Overview	2

3	Resource File	2

3.1	Alarm Play Intervals and Durations	3

3.2	Disabling Alarm Play Control	3

3.3	Alarm Play Sequence Repeat Option	3

4	Further Information	4

4.1	References	4

4.2	Open Issues	4

4.3	Glossary	4

# Introduction

## Purpose and Scope

This document is a guide to assist licensees in customising Alarm Server behaviour using the AlarmServer.rsc resource file.

Note: This document is primarily about AlarmServer.exe (in /common/generic/app-services/alarmserver/), not the Uikon Alarm Alert Server. The Uikon component shall be referred to as the “Alert Server” in this document.

# Overview

In Bravo, PREQ1118 adds support for an optional Alarm Server configuration resource file to customise the playing of alarm sounds. Prior to Bravo, the Alarm Server, via the Alert Server, instructed the licensee’s Alarm UI Application to turn on and off the alarm sound according to a fixed but API-modifiable sequence of intervals and durations. The new configuration support allows disabling this control altogether, or customising the sequence. If the resource file does not exist, or does not contain valid enteries, then the default behaviour will be maintained.

It is anticipated that additional uses for this configuration resource file may be supported in future.

# Resource File

The configuration file must be called alarmserver.rsc and must be in the alarmserver’s private directory (C:\private\101F5027 on emulator, Z:\private\101F5027 on target platforms.) It should start with the necessary include directives, resource name, and standard signature entry.

#include <badef.rh>
#include <alarmserver.rh>

NAME ASRV

RESOURCE BA_RSS_SIGNATURE
	{
	signature = 1;
	}

The resource file is only used for initial configuration. Alarm Server configuration is backed up to a file called AlarmServer.ini at runtime. That backup file takes precedence over the resource file the next time the Alarm Server is started. Therefore, if changes are made to the resource file, AlarmServer.ini should be deleted manually before restarting the Alarm Server.

## Alarm Play Intervals and Durations

Each entry in the sequence is a pair comprised of an offset and a duration. The offset is the time, in minutes, from the initial alarm time or the end of the last snooze. The duration refers to how long, in seconds, to play the alarm sound.

Here is an example that plays the sound for 30 seconds, then pauses for 30 seconds, and repeats that 2 more times:

RESOURCE sound_controller
	{
	intervals =
		{
		SOUND_INTERVAL { offset=0; duration=30; },
		SOUND_INTERVAL { offset=1; duration=30; },
		SOUND_INTERVAL { offset=2; duration=30; };
		};
	}

If an interval is or intervals are specified, then one of them must have an offset of zero.

## Disabling Alarm Play Control

This is a special case of “Alarm Play Interval Configuration” which is described in the previous section. Simply provide an empty interval list.

RESOURCE sound_controller
	{
	// disable alarm server control of alarm intervals
	intervals = {};
	}

This configuration cannot be overridden at runtime. RASCliSession::SetAlarmPlayIntervalsL may not be used to change this option, i.e. it cannot turn off alarm sound control by specifying an empty sequence, nor turn on alarm sound control by overriding an empty sequence in the resource file.

Durations that extend into the next interval cause the whole sequence to revert to the default hard-coded values – see sound settings source code.

## Alarm Play Sequence Repeat Option

When the end of the alarm play sequence is reached, the previous behaviour was to restart the sequence. So after a few hours the alarm would drop to playing once per hour, but then it would return to playing every minute again.

For backward compatibility, the above behaviour will still be the default if there is no resource file, or if the resource file does not specify an option. However, there are now three possible end-of-sequence options:

EAlarmSoundRepeatSettingStop       - Default if no resource file is present

EAlarmSoundRepeatSettingRepeatLast - Repeat last interval indefinitely

EAlarmSoundRepeatSettingLoop       - Stop when end of sequence reached

This option is an optional entry in the resource file. The syntax is as follows:

RESOURCE sound_controller
	{
	option = EAlarmSoundRepeatSettingRepeatLast;
	intervals = …

This option can not be overridden at runtime.

# Further Information

## References

| No. | Document Reference | Version | Description |
| --- | --- | --- | --- |
| [R1] | < SGL.GT0257.201> | 1.0 | PREQ1118 Design Sketch |

## Open Issues

The following issues need to be resolved before this document is completed;

- None.

## Glossary

The following technical terms and abbreviations are used within this document.

| Term | Definition |
| --- | --- |