Contents

1	Introduction	4

1.1	Purpose and Scope	4

2	Overview	4

3	HCI migration [required]	4

3.1	HCIv1	4

3.2	HCIv2 (Bluetooth v2.0)	4

4	User Interface Notifiers [required]	5

4.1	New Notifier Types	5

4.1.1	Numeric Comparison	5

4.1.2	Passkey Entry	5

4.1.3	PIN Code Entry [optional]	5

4.2	Notifier Implementation	6

4.2.1	Numeric Comparison	7

4.2.2	Passkey Entry	8

4.2.3	PIN Code Entry	9

5	Initiating Dedicated Bonding [recommended]	10

5.1	Connecting to the Pairing Server	10

5.2	Initiating a Dedicated Bond	10

5.3	Cancelling a Dedicated Bond Request	11

6	Service Security Settings [optional]	11

6.1	Basic Usage	11

6.2	Security API Migration	12

6.2.1	Migration to new APIs	12

6.2.2	Migration guidance	14

6.2.3	Summary	15

6.3	Legacy API Behaviour	15

6.3.1	Authentication Required	15

6.3.2	Authentication Not-required	15

6.3.3	Redundant functions	15

7	Bluetooth Extended Inquiry Information [optional]	16

7.1	Basic Usage	16

7.2	Migration to new Actions	17

7.2.1	Received Signal Strength Indication	17

7.2.2	Extended Inquiry Response Data	18

7.3	Legacy Actions Behaviour	20

7.4	Device selection notifier	20

8	Out-of-Band Data for Authentication [optional]	20

8.1	Connecting to the Pairing Server	20

8.2	Opening an OOB Data Session	20

8.3	Retrieving Local OOB Data	21

8.4	Refreshing Local OOB Data	21

8.5	Providing OOB Data from Remote Devices	21

8.6	Invalidating OOB Data from Remote Devices	22

9	Manufacturer Specific Data in Inquiry Responses [optional]	22

9.1	Creating the Publisher	22

9.2	Publishing New Data	22

9.3	Handling the Data Request Call-back	23

10	Physical Link Metrics [optional]	23

10.1	Enabling Physical Link Metrics	23

10.2	Subscribing to Metrics	24

10.3	Receiving Updates	24

11	Simple Pairing Debug Mode [optional]	24

11.1	Enabling Simple Pairing Debug Mode	25

12	Further Information	26

12.1	References	26

12.2	Open Issues	26

12.3	Glossary	26

# Introduction

## Purpose and Scope

This document provides guidance to a licensee of Symbian OS who wishes to migrate their stack to support the mandatory features made available in Bluetooth v2.1.  The document also serves as a guide on how to use the new features with some examples.

This document is aimed at engineers/developers who are working with the Symbian OS Bluetooth stack for Bluetooth v2.1.  It is expected that the reader has a basic understanding of the core Bluetooth concepts, and some familiarity with the new features introduced in the v2.1 of the Bluetooth Core specification [R1].

From this point on the Symbian OS Bluetooth stack will be referred to as “the Bluetooth stack”.

# Overview

The tasks discussed below can be split roughly into three categories.

- Required: Tasks required for migration to the Bluetooth stack supporting Bluetooth v2.1.

- Recommended: Tasks which demonstrate new APIs, the use of which is optional, but failing to use them may results in a device having less supported features than with previous releases of the Bluetooth stack.

- Optional: Tasks which demonstrate new APIs, the use of which is optional, but their use may provide better user experiences.

In each case the task highlights which of these categories it falls under.  The order of the tasks flows roughly from the required tasks, to the more optional tasks.  As such this document is best read sequentially.

The example code snippets demonstrating the usage of APIs have been written with clarity as the main goal; as such the code may be missing “boiler plate code” needed when writing Symbian OS software, it may also not demonstrate best practice (for example User::WaitForRequest is typically used where the Active Object pattern would be better suited).

# HCI migration [required]

The version of the Bluetooth stack that supports Bluetooth v2.1 relies on version 2 of the Symbian OS HCI interface (hereafter referred to as HCIv2).  As a licensee may have migrated to HCIv2 before taking the new stack there are two possible migration paths.

- HCIv1 -> HCIv2 (v2.1)

- HCIv2 (v2.0) -> HCIv2 (v2.1)

## HCIv1

If it is the first migration path that is appropriate then the complete details for migration are beyond the scope of this document.  Please consult the HCIv2 migration how-to document [R2]. You may wish to return to this document to cover the v2.1 specific changes to the HCI at a later date.

## HCIv2 (Bluetooth v2.0)

For the second migration path there is little that is required to support the Bluetooth v2.1 features due to the nature of HCIv2.

With the addition of new commands and events to the Bluetooth v2.1 Core specification, new commands and events have been added to the CommandsEvents static DLL interface as new ordinals in the exported DEF file.  If a licensee has branched the CommandsEvents implementation from the reference Symbian implementation, they will need to update the implementation to include all the new Bluetooth v2.1 commands and events.

Otherwise all HCIv2 interfaces are agnostic to Bluetooth v2.0 and Bluetooth v2.1 – of course consideration should be made to hardware changes with the Bluetooth controller which may require modifications to the actual implementation behind these interfaces.

# User Interface Notifiers [required]

In addition to PIN Code Entry, Bluetooth v2.1 introduces four additional authentication mechanisms.  Two of these additional methods require user interaction, and as such information needs to be communicated to and from the user of the device.

The Bluetooth stack uses the Symbian OS Notifier framework to fulfil this UI requirement; in addition to the original PIN Code Entry and authorisation notifiers, three new notifier types have been added.

## New Notifier Types

### Numeric Comparison

Numeric Comparison involves the user comparing 6-digit numbers displayed on both devices that are authenticating.  Authentication is completed by the users of both devices signalling a confirmation that the displayed numbers are identical – if one device is unable to input a confirmation then only the other device needs to signal confirmation.

The UID representing a numeric comparison notifier is KBTNumericComparisonNotifierUid.

The class that provides the parameters for the numeric comparison notifier is TBTNumericComparisonParams. The functions on the class provide access to the various details that the licensee may wish to use to present a suitable user interface to the device user.

- Numerical Value – the 6-digit number which the user is to confirm is identical on both devices.

- Comparison Scenario – an indication as to whether the user should confirm on just the Symbian OS device (i.e. the remote device has no input capabilities) or whether confirmation should be made on both Bluetooth devices.

- Locally Initiated – an indication as to whether the authentication was initiated by a service on the Symbian OS device.  A UI may wish to use this to indicate whether the authentication attempt was initiated by the user’s device, or whether it was triggered by the remote device.

### Passkey Entry

Authentication by Passkey Entry involves the Symbian OS device displaying a 6-digit number which is manually entered on a device without a display (but with a numeric keypad).  This is similar to PIN Code Entry except in this case the number is chosen for the user, and it need only be entered on the peer device.

To avoid confusion with the older PIN Code Entry notifier, the Symbian OS notifier is referred to as the Passkey Display notifier (as a Symbian OS device will always take on the display part of the authentication method).

The UID representing a passkey display notifier is KBTPasskeyDisplayNotifierUid.

The class that provides the parameters for the passkey display notifier is TBTPasskeyDisplayParams.  Like the numeric comparison notifier parameter class the details for a UI to use are available through the provided functions on the class.

- Numerical Value – the 6-digit number which the user is to enter on the numeric keyboard of the remote device.

- Locally Initiated – an indication as to whether the authentication was initiated by a service on the Symbian OS device. A UI may wish to use this to indicate whether the authentication attempt was initiated by the user’s device, or whether it was triggered by the remote device.

### PIN Code Entry [optional]

Although a PIN Code Entry notifier was already required for the Bluetooth stack prior to Bluetooth v2.1, it lacked future proof extensions.  As an extension was needed to provide more information for the user interface a new notifier has been created.  This notifier is only used for Bluetooth v2.0 PIN code entry use-cases.

The new PIN Code Entry notifier is incompatible with the old notifier, however if not present the stack will invoke the old PIN Code Entry notifier so migration is not necessary unless the additional information provided in the parameters of the new notifier are required.

The UID representing the new PIN code entry notifier is KBTPinCodeEntryNotifierUid.

The class that provides the parameters for the PIN code entry notifier is TBTPinCodeEntryNotifierParams.  Following the new format for notifier parameters, the details are access through functions instead of directly as data members.

- PIN Code Minimum Length – the minimum number of PIN code digits that must be entered by the user.

- Locally Initiated – an indication as to whether the authentication was initiated by a service on the Symbian OS device.  A UI may wish to use this to indicate whether the authentication attempt was initiated by the user’s device, or whether it was triggered by the remote device.

- Strong PIN Code Required – an indication as to whether a service has specified a strong level of security.

#### Strong PIN Code Required

The extension to the PIN Code Entry notifier is the communication of a strong PIN code requirement.  This additional information has been provided to allow the UI to attempt to preserve the security requirements of 2.1 aware services when connecting to legacy (pre-v2.1) Bluetooth devices.  The Boolean value will be true when a service that is being connected has “MITM protection required”.

When connecting with legacy devices PIN Code Entry is the only authentication method available and the strength of the authentication (read link key) is related to the length of the PIN code used.  The Bluetooth core specification v2.1 [R1] indicates in Volume 3 Part C Table 5.7 recommends that a 16 digit PIN code be used when a service requiring MITM protection is being connected to by a legacy device.

It is expected that this field should be used by the UI to prompt the user into providing a strong PIN code, or perhaps to suggest a strong PIN code for the user to enter.

It is recommended that the UI does not mandate a PIN code for a user to enter, nor force the length of a PIN code the user must enter (if a profile specifies a PIN code length then the minimum PIN code length field will indicate this).  The reason for this recommendation is that such policies may harm interoperability with legacy devices (especially devices that provide a fixed PIN).

#### Disadvantages of Not Migrating

If the new PIN code entry notifier is not available, the Bluetooth stack will invoke the previous PIN code entry notifier.  The disadvantage of not migrating is that the UI cannot communicate to the user that they should use a strong PIN code because of the service MITM protection requirements.

As such if a strong PIN code is required the stack will communicate this to the old notifier by specifying a minimum PIN code length of 16 digits.  This has the disadvantage that the user will be forced to enter a 16-digit PIN without any reason, and it will be impossible to interoperate with devices that have fixed PIN codes of shorter length.

For these reasons it is strongly recommended that licensees migrate to the new PIN code entry notifier.

## Notifier Implementation

The details on how to implement a Symbian OS notifier are documented in various other sources depending on the notifier framework that is to be used.  This document provides an illustration of how to use the data provided through the general notifier framework interface.

### Numeric Comparison

#### Start-up

The start of the numeric comparison notifier provides most of the information the UI needs to communicate to the user.

#include <btextnotifiers.h>

void CNumericComparisonNotifier::StartL(

const TDesC8& aBuffer,

TInt aReplySlot,

const RMessagePtr2& aMessage)

{

TBTNumericComparisonParamsPckg params;

params.Copy(aBuffer);

TBTDevAddr addr = params().DeviceAddress();

TPtrC name = params().DeviceName();

TUint32 number = params().NumericalValue();

TComparisonScenario compScen = params().ComparisonScenario();

TBool local = params().LocallyInitiated();

TRequestStatus status;

TBool compares = EFalse;

DisplayAndGetResponse(addr, name, number, compScen, local, compares, status);

User::WaitForRequest(status);

TInt error = status.Int();

if(error == KErrNone)

{

error = aMessage.Write(aReplySlot, TPckgC<TBool>(compares));

}

aMessage.Complete(error);

}

#### Updates

Currently only one type of update is available for the numeric comparison notifier, an update to the device name of the remote device to which the pairing is being performed with.

:

:

void CNumericComparisonNotifier::UpdateL(

const TDesC8& aBuffer,

TInt /*aReplySlot*/,

const RMessagePtr2& aMessage)

{

TBTNotifierUpdateParamsPckg2 pckgRaw;

pckgRaw.Copy(aBuffer.Left(pckgRaw.MaxLength()));

if (pckgRaw().Type() == TBTNotifierUpdateParams2::EDeviceName)

{

TBTDeviceNameUpdateParamsPckg namePckg;

namePckg.Copy(aBuffer);

if (namePckg.Result() == KErrNone)

{

TPtrC newName = namePckg().DeviceName();

TRequestStatus status;

DisplayNewName(newName, status);

User::WaitForRequest(status);

aMessage.Complete(status.Int());

}

else

{

aMessage.Complete(KErrNone);

}

}

else

{

// Ensure unrecognised types are indicated as not supported for

// future compatibility.

aMessage.Complete(KErrNotSupported);

}

}

### Passkey Entry

#### Start-up

The data provided in the start of the passkey entry display notifier provides the main information to communicate to the user what needs to be done.

It is worth noting that the display side of a passkey entry association is unable to explicitly abort a pairing attempt.  The notifier can be completed to dismiss it, however the pairing process will remain ongoing internally until it is completed by the keyboard or the pairing procedure times out.

#include <btextnotifiers.h>

void CPasskeyDisplayNotifier::StartL(

const TDesC8& aBuffer,

TInt aReplySlot,

const RMessagePtr2& aMessage)

{

TBTPasskeyDisplayParamsPckg params;

params.Copy(aBuffer);

TBTDevAddr addr = params().DeviceAddress();

TPtrC name = params().DeviceName();

TUint32 number = params().NumericalValue();

TBool local = params().LocallyInitiated();

TRequestStatus status;

DisplayAndWaitForDismiss(addr, name, number, compScen, local, status);

User::WaitForRequest(status);

aMessage.Complete(status.Int());

}

#### Updates

The passkey entry display notifier can currently receive two types of updates.

- An update to the remote device name (the same as the other notifiers).

- An update to provide the notification of a key press by the remote keyboard.

As a Symbian OS device will be the display part of the pairing method, the remote device will be keyboard only and so not feature a display.  It is highly recommended that the key press notifications provide a visual representation of keys being registered to help the user.

:

:

void CPasskeyDisplayNotifier::UpdateL(

const TDesC8& aBuffer,

TInt /*aReplySlot*/,

const RMessagePtr2& aMessage)

{

TBTNotifierUpdateParamsPckg2 pckgRaw;

pckgRaw.Copy(aBuffer.Left(pckgRaw.MaxLength()));

if (pckgRaw().Type() == TBTNotifierUpdateParams2::EDeviceName)

{

TBTDeviceNameUpdateParamsPckg namePckg;

namePckg.Copy(aBuffer);

if (namePckg.Result() == KErrNone)

{

TPtrC newName = namePckg().DeviceName();

TRequestStatus status;

DisplayNewName(newName, status);

User::WaitForRequest(status);

aMessage.Complete(status.Int());

}

else

{

aMessage.Complete(KErrNone);

}

}

else if (pckgRaw().Type() == TBTNotifierUpdateParams2::EPasskeyDisplay)

{

TBTPasskeyDisplayUpdateParamsPckg pckg;

pckg.Copy(aBuffer);

THCIPasskeyEntryNotificationType keypress =

pckg().KeypressNotification();

TRequestStatus status;

UpdateKeypressDisplay(keypress, status);

User::WaitForRequest(status);

aMessage.Complete(status.Int());

}

else

{

// Ensure unrecognised types are indicated as not supported for

// future compatibility.

aMessage.Complete(KErrNotSupported);

}

}

### PIN Code Entry

#### Start-up

The start up of the new PIN code entry notifier should be very similar to the previous PIN code entry notifier.

#include <btextnotifiers.h>

void CPinCodeEntryNotifier::StartL(

const TDesC8& aBuffer,

TInt aReplySlot,

const RMessagePtr2& aMessage)

{

TBTPinCodeEntryNotifierParamsPckg params;

params.Copy(aBuffer);

TBTDevAddr addr = params().DeviceAddress();

TPtrC name = params().DeviceName();

TUint minLength = params().PinCodeMinLength();

TBool local = params().LocallyInitiated();

TBool strong = params().StrongPinCodeRequired();

TRequestStatus status;

TBTPinCode pin;

DisplayAndGetPin(addr, name, minLength, local, strong, pin, status);

User::WaitForRequest(status);

TInt error = status.Int();

if(error == KErrNone)

{

error = aMessage.Write(aReplySlot, pin);

}

aMessage.Complete(error);

}

#### Updates

The PIN code entry notifier currently only receives one type of update, that being an update to the device name of the remote device with which pairing is proceeding.

An example of how this may be implemented is illustrated in the numeric comparison notifier example (§4.2.1.2).

# Initiating Dedicated Bonding [recommended]

Before Symbian OS supported Bluetooth v2.1 no dedicated bonding API was available to licensees.  Often this process would be simulated by creating a connection to a well known service (such as SDP) such that the outgoing connection required authentication.

Bluetooth v2.1 introduces a stage in the authentication process where by each device communicates what the type of bonding that is intended for the result of a pairing.  As such it is necessary for the stack to know when it is initiating a dedicated bonding attempt as distinct from a general bonding performed as part of a connection establishment.

For this reason a separate dedicated bonding API has been provided to follow the dedicated bonding procedure.  It is optional to use this new API, however if not used then the Symbian device will not pass the GAP qualification for initiating dedicated bonding.

Additionally, use of this API is recommended for usability reasons – the use of the bonding intent will provide an enhanced Bluetooth experience for the user.

## Connecting to the Pairing Server

The dedicated bonding API is handled through the pairing server introduced for Bluetooth v2.1.  As such the first step is to connect a session to the pairing server.

#include <bluetooth/pairing.h>

:

:

RBluetoothPairingServer pairingServer;

TInt error = pairingServer.Connect();

if(error != KErrNone)

{

return error;

}

## Initiating a Dedicated Bond

With a session to the pairing server established a dedicated bond can be initiated through the RBluetoothDedicatedBondingInitiator class.

RBluetoothDedicatedBondingInitiator dedicatedBonder;

TBTDevAddr devAddr(/*the device address of device to bond with*/);

TRequestStatus status;

dedicatedBonder.Start(pairingServer, devAddr, status);

User::WaitForRequest(status);

dedicatedBonder.Close(); // finished with session

if(status.Int() != KErrNone)

{

return status.Int();

}

The class does not form a traditional session like API; this is because the class represents an individual dedicated bonding session to the remote device.  As such the start function creates the session and initiates the dedicated bonding.  Once completed the session is over and thus should be closed.

## Cancelling a Dedicated Bond Request

An outstanding dedicated bonding request can be cancelled by closing the RBluetoothDedicatedBondingInitiator handle which issued the request.

:

:

dedicatedBonder.Close();

User::WaitForRequest(status);

# Service Security Settings [optional]

The Bluetooth service security settings have been updated to provide direct control for secure simple pairing, notably the inclusion of specification for Man-in-the-Middle protection levels.

## Basic Usage

The Symbian OS Bluetooth security settings are originally based on the parameters to security mode 2 from the Bluetooth Core specification Volume 3 Part C Section 5.2.1.2 [R1].

The main security APIs for the Bluetooth stack are encapsulated in the TBTAccessRequirements and TBTServiceSecuirty classes.  TBTServiceSecurity is essentially the aggregation of a service UID and an instance of the TBTAccessRequirements; the security APIs TBTServiceSecurity exposes can be considered as forwarding calls to the TBTAccessRequirement instance.  As such discussion of API migration will focus solely on the TBTServiceSecurity class since the discussion will be identical for users of TBTAccessRequirements.

The service requirements for a connection is done by using the TBTServiceSecurity class to specify the security requirements for the service, and then setting that value in the TBTSockAddr used for the service – be that either for initiating a connection to a remote device, or binding to a listening socket.

#include <bt_sock.h>

:

:

// Establish socket server session

RSocketServ socketServ;

User::LeaveIfError(socketServ.Connect());

:

:

// Connect to a remote device

RSocket initiator;

User::LeaveIfError(initiator.Open(socketServ, _L("L2CAP")));

CleanupClosePushL(initiator);

TBTServiceSecurity sec;

sec.SetAuthentication(/*Authentication Requirement*/);

sec.SetEncryption(/*Encryption Requirement*/);

sec.SetAuthorisation(/*Authorisation Requirement*/);

sec.SetUid(/*Symbian UID identifying service*/);

TBTDevAddr devAddr(/*Address to remote device*/);

TBTSockAddr sockAddr;

sockAddr.SetBTAddr(devAddr);

sockAddr.SetPort(/*Remote service to connect to*/);

sockAddr.SetSecurity(sec);

TRequestStatus status;

initiator.Connect(sockAddr, status);

User::WaitForRequest(status);

:

:

// Create a listenning service

RSocket listener;

User::LeaveIfError(listener.Open(socketServ, _L("L2CAP")));

CleanupClosePushL(listener);

TBTServiceSecurity sec;

sec.SetAuthentication(/*Authentication Requirement*/);

sec.SetEncryption(/*Encryption Requirement*/);

sec.SetAuthorisation(/*Authorisation Requirement*/);

sec.SetUid(/*Symbian UID identifying service*/);

TBTSockAddr sockAddr;

sockAddr.SetPort(/*Local service port to listen on*/);

sockAddr.SetSecurity(sec);

User::LeaveIfError(listener.Bind(sockAddr));

User::LeaveIfError(listener.Listen(/*Number of incoming connections to queue*/));

RSocket accepted;

TRequestStatus status;

listener.Accept(accepted, status);

User::WaitForRequest(status);

:

:

How to use Bluetooth sockets on Symbian OS is well documented in the Symbian OS library and through example code.  As such this document will focus on the security aspects introduced with Bluetooth v2.1.

## Security API Migration

### Migration to new APIs

The new APIs that have been introduced to support Bluetooth v2.1 are illustrated in the class declaration below in the colour blue.  APIs which should be migrated from are highlighted in the colour red.

// from bt_sock.h

:

:

/**

Indicates the level of Man-In-The-Middle protection that is required

for a Bluetooth connection.

*/

enum TBluetoothMitmProtection

{

EMitmNotRequired = 0x0, //!< MITM protection is not required

EMitmDesired = 0x1,	//!< MITM protection will be used if possible

EMitmRequired = 0x2	//!< MITM protection is required

};

:

:

NONSHARABLE_CLASS(TBTServiceSecurity)

/** The security settings of a bluetooth service.

Contains information regarding the service UID and the access requirements.

@publishedAll

@released

*/

{

public:

IMPORT_C TBTServiceSecurity(const TBTServiceSecurity& aService);

IMPORT_C TBTServiceSecurity();

IMPORT_C void SetUid(TUid aUid);

IMPORT_C void SetAuthentication(TBluetoothMitmProtection aPreference);

IMPORT_C void SetAuthentication(TBool aPreference);

IMPORT_C void SetAuthorisation(TBool aPreference);

IMPORT_C void SetEncryption(TBool aPreference);

IMPORT_C void SetDenied(TBool aPreference);

IMPORT_C TInt SetPasskeyMinLength(TUint aPasskeyMinLength);

IMPORT_C TBool AuthorisationRequired() const;

IMPORT_C TBool EncryptionRequired() const;

IMPORT_C TBool AuthenticationRequired() const;

IMPORT_C TBool Denied() const;

IMPORT_C TUint PasskeyMinLength() const;

IMPORT_C TBluetoothMitmProtection MitmProtection() const;

IMPORT_C TUid Uid() const;

};

:

:

The specification of authentication, authorisation and encryption for the security of a Bluetooth service is defined in the General Access Profile (GAP) of the Bluetooth Core specification for when a device is operating in security mode 2. With Bluetooth v2.1 the existing security modes (1, 2 and 3) have been deprecated and replaced by security mode 4.  In security mode 4 a set of security levels (0 to 3) have been defined, going from no security (level 0) to high security (level 3).

For compatibility reasons the Symbian Bluetooth security APIs have remained mostly the same, a service will continue to specify user authorisation if confirmation for the use of a service is required.  Similarly a service will continue to specify if the link over which it is to operate should be encrypted.

For the case of authentication, Bluetooth v2.1 has resulted in a change in the meaning of “authentication”. Previously (Bluetooth v2.0 and earlier) authentication would result in a one-time PIN code entry “pairing” process that was to be completed by both devices between which authentication was taking place.  For Bluetooth v2.1 authentication can be performed with 5 different pairing mechanisms, some of which do not actually provide authentication in the traditional sense.

As such the specification of whether the link should be authenticated or not needs to be migrated to a richer definition of authentication requirements in terms of protection against Man-In-The-Middle (MITM) attacks.  The introduction of the TBluetoothMitmProtection enumeration provides the values that a developer can use for the specification of these requirements.  It is with this richer definition that the Symbian OS security API can reflect the security levels mandated by the Bluetooth v2.1 specification.

| Man-In-The-Middle Protection Level | Description |
| --- | --- |
| Not Required | This specifies that the application does not require man-in-the-middle protection. As such if the remote device also does not require MITM protection then an unauthenticated link key will be established between the devices. This is equivalent to security level 2 when operating in security mode 4. |
| Desired | This specifies that the application would like man-in-the-middle protection. Therefore if the remote device is capable of performing numeric comparison or passkey entry association they will be initiated. If the device is not capable of generating an authenticated link key an unauthenticated link key will be created. |
| Required | This specifies that the application requires man-in-the-middle protection. Here if the remote device is not capable of performing numeric comparison or passkey entry then the connection attempt will fail. This is equivalent to security level 3 when operating in security mode 4. |

Table 1 - Man-In-The-Middle Protection Levels

The Bluetooth specification discusses MITM protection in terms of only two levels – “required” and “not required” – the Symbian API provides “desired” in addition to allow applications to attempt to achieve the best security available for the link without compromising interoperability.

### Migration guidance

The migration to the specification of MITM protection levels needs to be made carefully with considerations to the use-cases for a particular Bluetooth service.  To help developers make the decision a consideration of the effect each of the MITM protection levels generates is made within a different context.

The initial considerations focus solely on Bluetooth v2.1 devices, consideration of pre-v2.1 hardware/devices is made in its own section.

It is also worth noting that these considerations are about impact of the pairing process.  When devices are bonding then pairing is a one-time process.  Therefore the developer should remember that these issues are not appropriate once devices have been bonded.

#### Security

With MITM protection not-required, the result is that the link to a particular device is not guaranteed to be the device you believe it is.  It is possible for a malicious device to “spoof” the remote device and become a Man-In-The-Middle. As a result any data that is transferred over the compromised link can be intercepted by the malicious device.

With MITM protection required it is cryptographically very difficult for a malicious device to be able to intercept data transferred between two associated devices.  As such it is the most secure of protection levels.

With MITM protection desired you may appear to get a “half-way house” between not-required and required the MITM protection will be available, however this is not the case.  The security of this protection level can be considered almost equivalent to the MITM not-required as a malicious remote device could “spoof” the input/output capabilities it possesses and so force no MITM protection.  However, this level should be considered more secure than MITM protection not-required since if the actual remote device is capable of performing MITM-protected pairing then the bonding will be sufficiently strong enough to prevent any “spoofing” attacks made by a malicious device at a later time.

#### Ease of use

With MITM protection not-required the user of a device will not need to interact with the device at all – as a result the use of the service very straight forward.

With MITM protection required, the user of a device will always be required to go through a pairing procedure the first time a service requiring MITM protection is required.  Although this is not necessarily a difficult process, the more user interaction required by the user of the device the less easy that particular service could be considered to use.

With MITM protection desired the usability is dependant on the capabilities of the remote device.  This could be considered to be less usable than MITM protection required since the user interactions required for establishing a connection could be considered more deterministic.

#### Interoperability

With MITM protection not-required as no input is required then there will be no interoperability problems.

With MITM protection required there will be a reduction of interoperability.  If a remote device has insufficient input/output capabilities for providing MITM protection then the service will not be able to connect/interoperate with that device.

With MITM protection desired then the Bluetooth stack will adapt to the capabilities of the remote device and so is as interoperable as MITM protection not-required.

#### Legacy (pre-v2.1) hardware

The use of legacy hardware will remove the security provided to guard against malicious devices sniffing the authentication procedure between two legitimately authenticating devices.  However the new APIs do map onto the old pairing scheme.  The original (pre v2.1) authentication scheme implicitly used man-in-the-middle protection – the input of the same PIN code to both devices indicated that the two devices involved in the pairing are the actual devices the user was inputting the PIN codes to.

With MITM protection not-required no PIN code entry will be performed unless encryption is required by the service.

With MITM protection desired or required then PIN code entry will be performed.

### Summary

The bulleted list below provides a simplified summary to section 6.2.2, providing the general trends for selecting MITM protection levels.

- If minimal user interaction (ease of use) is required at the expense of security – MITM protection not-required should be used.

- If strong security is absolutely required at the expense of ease of use and interoperability – MITM protection required should be used.

- In all other cases MITM protection desired should be strongly considered for use.

## Legacy API Behaviour

This section considers what can be expected when using the legacy Bluetooth security APIs.  By not selecting a particular level of MITM protection the Bluetooth stack attempts to perform a “best effort” security specification for a particular service.  As such the semantics of the legacy security specification might not match the desired security requirements of a particular service.  For this reason migration to explicit specification of MITM protection requirements is strongly recommended.

### Authentication Required

If authentication is required, this is interpreted as a service not being bothered about user interaction (since for pre-v2.1 authentication required PIN code entry).  Additionally interoperability is favoured over security, the reason being that the Bluetooth v2.1 provides greater security than pre-v2.1 had anyway.

Therefore MITM protection desired is generally the resulting MITM protection requirement.

The exceptional case is if the service has specified a minimum PIN code length for the service.  If a service has specified the maximum PIN code length (16 digits) the Bluetooth stack interprets this as a strong requirement for security.  In such cases MITM protection required is the resulting setting.

### Authentication Not-required

If authentication is not required, this indicates that the service would trade off security for ease of use.  Following the analysis in section 2.1.1 the resulting MITM protection requirement is MITM protection not required.

### Redundant functions

The TBTServiceSecurity::AuthenticationRequired function is no longer needed once migrated to the new MITM protection specification API.  The MITM protection requirement can be determined by the TBTServiceSecurity::MitmProtection function.

The TBTServiceSecurity::AuthenticationRequired function will continue to return whether authentication will be required on the link.

# Bluetooth Extended Inquiry Information [optional]

Bluetooth v2.1 introduced extended inquiry responses (EIR) to provide additional information for the recipient to use.  Some of the types of information potentially available in an extended inquiry response are defined in the Bluetooth Core specification [R1].

The Symbian API for initiating inquiries has been extended to provide access to this richer set of information from an inquiry.  Additionally the API has been extended to provide the RSSI for the inquiry result.

## Basic Usage

The Symbian OS Bluetooth inquiry API uses the host resolver framework provided by the socket server.  Inquiries and name requests (for both local and remote devices) are initiated through it.  The specification of what is to be initiated is based on both the function call made, and for some cases the set of specified “actions”.

#include <bt_sock.h>

:

:

// Establish socket server session

RSocketServ socketServ;

User::LeaveIfError(socketServ.Connect());

// Find the link manager protocol layer

TProtocolDesc protocolInfo;

User::LeaveIfError(iSocketServer.FindProtocol(_L("BTLinkManager"), protocolInfo));

// Open the host resolver

RHostResolver hostResolver;

User::LeaveIfError(hostResolver.Open(	socketServ,

protocolInfo.iAddrFamily,

protocolInfo.iProtocol));

:

:

// Get the local Bluetooth device name

TBTDeviceName localDeviceName;

hostResolver.GetHostName(localDeviceName);

:

:

// Perform a Bluetooth inquiry.

TRequestStatus status;

TNameEntry nameEntry;

TInquirySockAddr inqSockAddr;

TUint action;

TBool namesWanted = /* If the remote device name is wanted in the inquiry result */;

action = KHostResInquiry;

if(namesWanted)

{

action |= KHostResName;

}

inqSockAddr.SetAction(action);

inqSockAddr.SetIAC(/* Inquiry access code to use */);

TBool finished = EFalse;

hostResolver.GetByAddress(inqSockAddr, nameEntry, status);

do

{

User::WaitForRequest(status);

if(status.Int() == KErrNone

{

TInquirySockAddr& sa = TInquirySockAddr::Cast(nameEntry().iAddr);

TBTDevAddr deviceAddress = sa.BTAddr();

if(namesWanted)

{

THostName deviceName = nameEntry().iName;

}

hostResolver.Next(nameEntry, status);

}

else

{

// An successful end to an inquiry will be indicated with the

// KErrEof error code

finished = ETrue;

}

}

while(!finished);

:

:

// Get a remote Bluetooth device name

TInquirySockAddr nameSockAddr;

nameSockAddr.SetAction(KHostResName);

nameSockAddr.SetBTAddr(/*Bluetooth device address to get the name for*/);

hostResolver.GetByAddress(nameSockAddr, nameEntry, status);

User::WaitForRequest(status);

if(status.Int() == KErrNone)

{

TPtrC remoteDeviceName(nameEntry().iName);

}

:

:

hostResolver.Close();

socketServ.Close();

How to perform Bluetooth inquries on Symbian OS is documented in the Symbian OS library and through example code.  As such this document will focus on the extended inquiry aspects introduced with Bluetooth v2.1.

## Migration to new Actions

### Received Signal Strength Indication

The received signal strength indication (RSSI) for Bluetooth inquiry results has been available since Bluetooth v2.0, however no API was provided for this in Symbian OS until support for EIR was introduced.

The RSSI value can be accessed from the TInquirySockAddr structure used for an inquiry.

:

:

// RHostResolver hostResolver has already been opened

// Set-up the inquiry

TInquirySockAddr rssiSockAddr;

rssiSockAddr.SetAction(KhostResInquiry);

rssiSockAddr.SetIAC(/*inquiry access code to perform inquiry on*/);

TNameEntry nameEntry;

// Start the inquiry

TRequestStatus status;

hostResolver.GetByAddress(rssiSockAddr, nameEntry, status);

User::WaitForRequest(status);

User::LeaveIfError(status.Int());

// Get the RSSI value

TInquirySockAddr& sa = TInquirySockAddr::Cast(iResult().iAddr);

if(sa.ResultFlags() & TInquirySockAddr::ERssiValid)

{

TInt8 rssi = sa.Rssi();

}

hostResolver.Next(nameEntry, status);

User::WaitForRequest(status);

// Process next result

:

:

A potential use of the inquiry RSSI value is outlined in section 7.2.2.3.

### Extended Inquiry Response Data

To access EIR information for an inquiry result the KHostResEir action must be used.  This triggers the stack to encode the EIR data into the TNameRecord::iName member provided – meaning that the device name can no longer be read directly out of the field.

Instead the TBluetoothNameRecordWrapper should be used to extract the information returned for a particular inquiry result.

:

:

// RHostResolver hostResolver has already been opened

// Set-up the inquiry

TInquirySockAddr inqSockAddr;

TUint action = KHostResInquiry | KHostResEir;

inqSockAddr.SetAction(action);

inqSockAddr.SetIAC(/*Inquiry access code to perform inquiry on*/);

TNameEntry nameEntry;

TBluetoothNameRecordWrapper entryWrapper(nameEntry());

// Start the inquiry

TRequestStatus status;

hostResolver.GetByAddress(inqSockAddr, nameEntry, status);

User::WaitForRequest(status);

User::LeaveIfError(status.Int());

// Extract the device name

TInt deviceNameLength = entryWrapper.GetDeviceNameLength();

RBuf deviceName;

TBool nameIsComplete;

deviceName.CreateL(deviceNameLength);

TInt error = entryWrapper.GetDeviceName(deviceName, nameIsComplete);

TBool nameIsAvailable = (error == KErrNone);

// Get TX power level

TInt8 txPowerLevel;

error = entryWrapper.GetTxPowerLevel(txPowerLevel);

TBool txIsAvailable = (error == KErrNone);

// Get Service Class UUIDs

RExtendedInquiryResponseUUIDContainer uuidContainer;

error = entryWrapper.GetServiceClassUuids(uuidContainer);

TBool serviceClassUuidsAreAvailable = (error == KErrNone);

// Get Manufacturer Specific Data

TInt vendorSpecificDataLength = entryWrapper.GetVendorSpecificDataLength();

RBuf8 vendorSpecifcData;

vendorSpecificData.CreateL(vendorSpecificDataLength);

error = entryWrapper.GetVendorSpecificData(vendorSpecificData);

TBool vendorSpecificDataIsAvailable = (error == KErrNone);

hostResolver.Next(nameEntry, status);

User::WaitForRequest(status);

// Process next result

:

:

All inquiry results need to be access through the TBluetoothNameRecordWrapper class when the KHostResEir action is set, even for results that have no EIR data.  As EIR is susceptible to corruption (and thus loss) the distinction between an extended inquiry response and a basic inquiry response is minimal, and so the API user will not be informed about whether an individual response is extended or basic.

#### Using the remote Bluetooth device name

The need for this information is typically to provide a user friendly UI for using Bluetooth.  The need for this information is already well established and easily understood.  The presence of the device name in EIR results in quicker “discovery”.

#### Using the service class UUIDs

For a discovery process the application typically wants to filter the results to devices that support a particular set of use cases.

Basic inquiry results return the device class of the Bluetooth device – which could be used to filter results, but this is undesirable since, for example, a Smartphone may be capable of Audio use-cases so filtering on AV devices would result in bad inter-operability.  Basic inquiry results also return the service class of a Bluetooth device – as a bit field of major service classes this could be used to perform filtering, but this also is not a perfect solution as a device may be capable of, for example, a particular audio use-case but not the video use-case that the remote use is looking to use.

A Bluetooth profile assigns a set of use cases with a particular service class (that usually define the role in the use cases a device can be).  Therefore filtering by service class is the correct way to filter devices to be displayed to the user for a particular operation.

The service classes available on a remote device can be determined by an SDP query, which requires a connection to the remote device.  Therefore, for the same reason having the device name in the extended inquiry response, the service class UUIDs in EIR make the discovery process quicker for when this correct filtering is used.

#### Using the transmission power level

The RSSI for an inquiry result provides an indication of how strong the received radio link to the remote side is.  This on its own is not very useful, as an inquiry result from a device with a powerful class 1 radio at some distance may have the same RSSI value as the inquiry result from a device in close proximity with a class 3 radio.

A more useful value is the path-loss for a signal.  This provides a rough approximation of how close a remote device is from the inquiry result.  Path-loss can be calculated using the RSSI and inquiry transmission power level from the remote device issuing the inquiry.

The UI could use the path-loss value to show to the user the approximate proximity of devices they are searching for, helping them to select the correct device (such as devices with fixed names, like headsets).

The calculation of path-loss is illustrated in the Bluetooth Core specification [R1].

## Legacy Actions Behaviour

The Bluetooth stack remains fully compatible with the older inquiry action combinations.  The inquiry action will perform an inquiry and return results without any names (even if available by EIR).  The name action will trigger an explicit remote name request.  The name action used in combination with the inquiry action will perform an inquiry and if the remote does not support EIR, or only provides a partial name via EIR a remote name request is made.  If a full name is available through EIR then no remote name request is needed and the stack will make this optimisation.

The caching actions retain the same semantics as they have done prior to the support of Bluetooth v2.1.

## Device selection notifier

The device selection notifier API provided in earlier releases of the Symbian OS Bluetooth stack has not been updated to return EIR info.  If an application wishes to retrieve EIR specific data the host resolver API should be used.

It is expected that the existing device selection notifier API is sufficient for most application – currently the API allows the specification of device class or service UUID filtering, and for the selected device name and address to be returned.

It is recommended that the implementation of the device selection notifier is updated to take advantage of the extra features EIR provides.

# Out-of-Band Data for Authentication [optional]

Out-of-Band (or OOB) data is an authentication mechanism introduced with the other new authentication processes in Bluetooth v2.1.  Here the authentication is made by having some other metaphor for association.  An obvious example is NFC, where the authentication is made by the proximity of the devices.  To support this, the Bluetooth Core specification v2.1 [R1] defines a set of procedures and a standard format for the OOB data to be transferred in (Volume 3 Part C Section 5.2.2.7).  The important values for authentication are the Simple Pairing Hash and Randomiser.

It should be noted that authentication using OOB data can be made with only one device having the others OOB data, it is not necessary for both devices to have each others current OOB data.

The Bluetooth stack provides an API for both retrieving local OOB data to be sent to a remote device and to provide OOB data received from a remote device.

## Connecting to the Pairing Server

The OOB data API uses a session to the pairing server.  For establishing this session see §5.1.

## Opening an OOB Data Session

The RBluetoothOobData class provides the APIs for working with Bluetooth OOB data.  Before they can be used, the instance must be opened.

RBluetoothOobData oobData;

TInt error = oobData.Open(pairingServer);

if(error != KErrNone)

{

return error;

}

## Retrieving Local OOB Data

The local OOB data can be retrieved by using the RBluetoothOobData::ReadLocalOobData function.  It is asynchronous as there may be a delay in retrieving the value from the Bluetooth controller, however the Bluetooth stack will cache the latest value retrieved from the hardware so subsequent calls will be quicker.

:

:

TRequestStatus status;

TBluetoothSimplePairingHash hash;

TBluetoothSimplePairingRandomizer randomiser;

oobData.ReadLocalOobData(hash, randomiser, status);

User::WaitForRequest(status);

if(status.Int() != KErrNone)

{

return status.Int();

}

// Now send hash and randomiser to remote device

:

:

Because the API is asynchronous a cancel function is provided.

:

:

oobData.CancelReadLocalOobData();

User::WaitForRequest(status);

The duration of validity of the OOB data retrieved from the Bluetooth controller is dependant on the hardware.  It should be expected though that local OOB data is invalid once the Bluetooth controller is reset or power cycled.

## Refreshing Local OOB Data

As OOB data returned from the hardware is cached by the stack, if a client wishes to invalidate the current local OOB data they may use the RBluetoothOobData::RefreshLocalOobData function.  The next call made to RBluetoothOobData::ReadLocalOobData will trigger the Bluetooth controller to generate a new set of OOB data thus invalidating the previous hash and randomiser values.

:

:

oobData.RefreshLocalOobData();

// Previous OOB data will still be valid until ReadLocalOobData is called.

:

:

## Providing OOB Data from Remote Devices

The Bluetooth stack provides three functions for the provision of OOB data.  One takes an 8-bit descriptor of the OOB data and parses it for the hash and randomiser values, the format of the data is expected to be that described in the Bluetooth Core specification [R1].  The other two functions take in one case the hash value, in the other both the hash and randomiser values directly.

The first function is intended for when the licensee does not wish to use any of the other information encoded in the OOB data block sent from the remote.

:

:

TPtrC8 remoteData(aDataFromRemote);

TInt error = oobData.ProvideRemoteOobData(remoteData);

if(error != KErrNone)

{

return error;

}

The remaining two functions are intended for use when a licensee has parsed the OOB data and so has already extracted the hash and potentially also the randomiser values.

:

:

TBTDevAddr addr = ParseForAddressL(remoteData);

TBluetoothSimplePairingHash hash = ParseForHashL(remoteData);

TBluetoothSimplePairingRandomizer randomiser;

TRAPD(error, randomiser = ParseForRandomizerL(remoteData));

if(error == KErrNone)

{

error = oobData.ProvideRemoteOobData(addr, hash, randomiser);

}

else

{

error = oobData.ProvideRemoteOobData(addr, hash);

}

User::LeaveIfError(error);

## Invalidating OOB Data from Remote Devices

It may be that a particular hash/randomiser combination from a remote device is no longer applicable.  This could be inferred from the OOB protocol used (i.e. the remote device indicates that the OOB data it has provided is now invalid), or it could be based on some policy implemented on the local device (i.e. OOB data is only valid for 1 minute).

It should be noted that once OOB data has been used by the stack it will be discarded.

:

:

TBTDevAddr addr(/*Address of remote device where OOB data is no longer valid*/);

oobData.ClearRemoteOobData(addr);

# Manufacturer Specific Data in Inquiry Responses [optional]

The Bluetooth stack publishes several bits of additional data for extended inquiry responses; these are gathered from various other places in the stack without any additional developer effort.

The extended inquiry response format allows the provision of manufacturer specific data, which cannot be automatically retrieved by the Bluetooth stack.  As such an API has been provided to allow a licensee to add manufacturer specific data.

## Creating the Publisher

To publish manufacturer specific data, a licensee must create an instance of the CEirPublisherManufacturerSpecificData class.  This class requires an implementation of the MManufacturerSpecificDataPublisherNotifier mixin; the semantics expected to be implemented by the mixin functions will be explained later.

#include <bluetooth/eirpublishermanufacturerspecificdata.h>

:

:

MManufacturerSpecificDataPublisherNotifier& callbackHandler =

static_cast<MManufacturerSpecificDataPublisherNotifier&>(aCallbackHandler);

CEirPublisherManufacturerSpecificData* publisher =

CEirPublisherManufacturerSpecificData::NewL(callbackHandler);

Once a manufacturer specific data publisher has been created, no others can be created; only a single publisher can exist at any time.

## Publishing New Data

Once data is available to be published, the publisher should be instructed to signal the amount of data it wishes to publish.  Whenever the data to be published changes, the CEirPublisherManufacturerSpecificData::RequestPublishDataSize function should be called to indicate a desire to update the EIR data (this is valid even if the size provided is the same as a previous call).

:

:

TPtrC8 data(aDataToPublish);

publisher->RequestPublishDataSize(data.Length());

Notice that the data to be published has not been provided to the Bluetooth stack yet, the data should be provided as part of a call-back from the publisher (§9.3).

## Handling the Data Request Call-back

Once a publisher has requested its desire to publish some data of a specific size it may receive a call-back requesting the data the publisher wishes to place in the EIR data block.

The Bluetooth stack goes to some lengths to balance how much room within the fixed size EIR data block a particular publisher may use based on how much data all the individual publishers of EIR data wish to publish.  As such the call-back indicates how much data the publisher may publish – this may be less than what they requested in the first place.

The publisher must decide what data they wish to publish based on the amount of room they have been allocated.  The approach taken here is dependant on the type of data that a manufacturer wishes to publish, this could be all-or-nothing, or it may be possible to publish a subset of the total data.

:

:

TManufacturerDataPublisherCallbackHandler::MmsdpnNotifyAvailableDataSize(TUint aAvailableDataSize)

{

TPtrC8 data(iDataToPublish);

// Here the actual data to publish must be decided, here we consider

// the data to be such that each individual byte is independent of each

// other.  It is valid to provide KNullDesC8 if no data can be published.

publisher->SetManufacturerSpecificData(data.Left(aAvailableDataSize));

}

# Physical Link Metrics [optional]

The Bluetooth stack now provides an API for retrieving various metrics of an active baseband link.  The metrics which are available are

- Received Signal Strength Indication (RSSI)

- Link Quality

- Failed Contact Count

- Transmission Power Level

A description of these values can be found in the Bluetooth Core specification [R1].

## Enabling Physical Link Metrics

To get access to the metrics for a particular physical link, an instance of the CBluetoothPhysicalLinkMetrics class needs to be created.  To do this the developer must provide a valid instance of the MBluetoothPhysicalLinkMetricsObserver mixin, in addition to an open RSocketServ handle.  The baseband link to receive the metrics for can be specified by either explicitly provide the Bluetooth device address for the remote device, or by providing an RSocket which represents a connected Bluetooth socket.

#include <bt_sock.h>

:

:

TBTDevAddr addr(/*Address of a remote device to which we are connected*/);

RsocketServ socketServ;

User::LeaveIfError(socketServ.Connect());

MBluetoothPhysicalLinkMetricsObserver& observer =

static_cast<MBluetoothPhysicalLinkMetricsObserver&>(aObserver);

CBluetoothPhysicalLinkMetrics* metrics =

CBluetoothPhysicalLinkMetrics::NewL(observer, socketServ, addr); // explicit

If there is no current baseband link to the remote device specified, the attempt to create the CBluetoothPhysicalLinkMetrics instance will leave.

## Subscribing to Metrics

Once created the various physical link metrics can be subscribed to by calling the subscription functions on the CBluetoothPhysicalLinkMetrics instance.

:

:

metrics->SubscribeRssi();

metrics->SubscribeLinkQuality();

metrics->SubscribeFailedContactCounter();

metrics->SubscribeTransmitPowerLevel();

Once subscribed updates to that particular metric will be received by the call-backs into the MBluetoothPhysicalLinkMetricsObserver observer provided when the CBluetoothPhysicalLinkMetrics instance was created.

To stop receiving updates the CBluetoothPhysicalLinkMetrics::Cancel function can be called.  This will cancel all subscriptions.  If there are several subscriptions active and only one requires cancelling, then the CBluetoothPhysicalLinkMetrics::Cancel function should be used and the subscriptions that are to “remain” should be re-subscribed.

:

:

metrics->Cancel();

## Receiving Updates

When a change to a particular metric occurs the MBluetoothPhysicalLinkMetricsObserver mixin instance has the appropriate function called on it.  This will happen when the subscription for a metric is called for the first time, and then subsequently when the value changes.  The reported values are made on a best-effort basis; the latency of updates to values is unbounded, and every change in value observed by the hardware may not be reported.

If an error occurs the MBluetoothPhysicalLinkMetricsObserver::MbplmoError function will be called.  If called all current subscriptions will be aborted, and an implementation may want try re-subscribing if the error is deemed to be transient.

Please note though that a disconnection of the physical link with which the metrics are subscribed to will be reported through this call-back and subsequent re-subscriptions will always fail.  Once a physical link has disconnected, the CBluetoothPhysicalLinkMetrics instance can no longer be successfully used.

# Simple Pairing Debug Mode [optional]

With the additional protection afforded by secure simple pairing, it is no longer possible to follow Bluetooth traffic with an air “sniffer” after the encryption has been enabled.  As logging air traffic is often required to debug issues, a simple pairing debug mode is available.

The Bluetooth specification indicates that only one device needs to enable simple pairing debug mode for an air sniffer to be used.  However for security reasons, the Bluetooth stack will prevent connections authenticated with debug mode link keys from being used when the Symbian OS Bluetooth stack does not have debug mode enabled.  Link keys generated in debug mode will not be stored by the Symbian OS device and so bonding cannot take place during debug mode.

## Enabling Simple Pairing Debug Mode

The mode is controlled with a new P&S key – KPropertyKeyBluetoothSetSimplePairingDebugMode – that when the value is set to true will trigger the request to enable debug mode.  The P&S key KPropertyKeyBluetoothGetSimplePairingDebugMode will change value to represent what actual debug mode state the Symbian OS Bluetooth stack is in.

#include <bt_subscribe.h>

:

:

RProperty getter;

TInt error = getter.Attach(KPropertyUidBluetoothCategory,

KPropertyKeyBluetoothGetSimplePairingDebugMode);

if(error != KErrNone)

{

return error;

}

TRequestStatus status;

getter.Subscribe(status);

// Cannot define the setter property here, the Bluetooth stack must define the

// property itself.

error = RProperty::Set(KPropertyUidBluetoothCategory,

KPropertyKeyBluetoothSetSimplePairingDebugMode,

ETrue);

if(error != KErrNone)

{

return error;

}

TInt value = EFalse;

do

{

User::WaitForRequest(status);

error = status.Int();

if(error == KErrNone)

{

error = getter.Get(value);

}

}

while(error == KErrNone && !(value || (getter.Subscribe(status), EFalse)));

Once enabled, the Bluetooth stack simple pairing debug mode cannot be disabled.  The mode will only be disabled if the Bluetooth stack is unloaded, or the controller has been reset.

With the security risks in enabling debug mode, the CommDD PlatSec capability is required.  It is expected that access to this interface is well controlled by the device manufacturer. Additionally, because of the security risk debug mode carries, it is expected that a device manufacturer will provide a noticeable (and potentially permanent) UI notification that debug mode is enabled.  The device manufacturer shall provide a means to enable debug mode in production devices, so that devices in the field with Bluetooth issues can be investigated – this enabler should be obscure as it is not intended for use by end users.

# Further Information

## References

| No. | Document Reference | Version | Description |
| --- | --- | --- | --- |
| [R1] |  | 2.1 | Bluetooth Core Specification |
| [R2] |  | 1.0 | HCI v2 Migration How-To Document |

## Open Issues

## Glossary

| Term | Definition |
| --- | --- |
| HCI | Host Controller Interface |
| OOB | Out-of-Band |
| GAP | General Access Profile |
| NFC | Near Field Communication |
| EIR | Extended Inquiry Response |
| MITM | Man-in-the-Middle (normally with reference to a security attack) |
| RSSI | Received Signal Strength Indication |
| UUID | Universally Unique Identifier |

For example the Uikon framework documentation for notifiers can be found in the Symbian OS developer library

» Symbian OS v9.5 » Symbian OS guide » Application Framework » Using Application Architecture (APPARC) » How To Port Guide: Notifiers

» Symbian OS v9.5 » Symbian OS guide » Application Framework » Uikon engineering documents » Correct Implementation of Notifiers

Initiating dedicated bonding is an optional GAP feature and so the device can still complete GAP qualification without being able to initiate dedicated bonding – however the device will be listed without the support for dedicated bonding.

This is one-time unless the user removes the stored bonding (which happens automatically) on one of the devices.  If removed then the “pairing” procedure must be enacted again to achieve an authenticated status.

Here discovery can be defined as the process of presenting the user with a Bluetooth device for them to select for use with a particular use case.

Correct because it does not limit interoperability, and it also provides the tightest level of filtering – the device will be able to take part in the general use-case the user is trying to operate (they still may not be able to do exactly what they wanted if that particular feature is optional for the profile under use – but at that point the that level of filtering becomes application specific).